THE UNIVERSITY OF ZAMBIA

SCHOOL OF ENGINEERING

DEPARTMENT OF CIVIL AND ENVIRONMENTAL ENGINEERING

CEE 3311 – FLUID MECHANICS

LAB No:

TITLE:

GROUP MEMBERS:

DATE DUE:

ATTENTION:

1

## HYDROSTATICS AND MANOM ETRY

CHIBESA WEZI- 16144617

EMMANUEL CHIWELE- 16134858

MAKO MUTOMA-16140389

AUGUSTINE KUNDA- 16140222

10/05/19

MR HANDIA

TENSILE TEST OF STEEL AND ALUMINIUM.

AIM:

-To determine the relationship between stress and strain.

-To determine the yield, tensile and rapture strengths.

-To determine the elastic modulus.

-To determine the elongation and reduction in cross-sectional area.

# IMPORTANCE

The parameters obtained from this test can be used to determine the stress-strain relation of a metal and such information is important for alloy development, design, quality control and comparison of different sets of metals.

# PROCEDURE

For this experiment, the initial lengths of the three different rods (aluminium rod, deformed rod and round bar made of steel) were measured as well as the gauge lengths. The specimen was then placed in the loading machine the extensometer was attached. Thereafter, the load was set reading to zero then a load was applied at a rate less than 690kPa/min continually until the rod broke and the final readings were taken.

# RESULTS

The following table shows the raw data obtained in the experiment:

|  | Aluminum | Steel (Deformed Bar) | Steel (Round Bar) |
| --- | --- | --- | --- |
| Initial Length (mm) | 500 | 500 | 500 |
| Final Length (mm | 615 |  |  |
| Initial Gage Length (mm) | 50 | 50 | 50 |
| Final Gage Length (mm) | 60 |  |  |
| Initial Diameter (mm) | 10 | 10 | 10 |
| Final Diameter (mm) | 3.10 |  |  |
| Initial Area (mm2) | 78.540 | 78.540 | 78.540 |
| Final Area (mm2) | 7.55 |  |  |

The following table shows the derived data after performing the tensile test on the respective specimens:

|  | Aluminum | Steel (Deformed Bar) | Steel (Round Bar) |
| --- | --- | --- | --- |
| Yield Strength (MPa) | 77.05 | 590.5 | 406.1 |
| Tensile Strength (MPa) | 112.671 | 683.64 | 592.2 |
| Rapture Strength (MPa) | 108.32 | 680.32 | 588.3 |
| Elasticity Modulus (GPa) | 14.5 | 62.9 | 93.7 |
| Elongation (mm) | 10 |  |  |
| %Reduction of Area | 90.4 |  |  |

# DATA ANALYSIS

## STRESS-STRAIN CURVES

- ALUMINIUM:

- HIGH YIELD STEEL (DEFORMED BAR)

- MILD STEEL (ROUND BAR):

# DISCUSSION

# From the results, it was seen that the deformed bar had the highest tensile strength proving that it is able to withstand a higher load compared to the other two. It was also observed that the round steel bar had the lowest elongation showing that is more brittle. Aluminum had the highest area reduction which means its necking diameter was very small causing a huge reduction in area and it is ductile. The relationship between strain and stress was also determined; stress is directly proportional to strain provided it is within the proportion limit and the elastic modulus is kept constant.

# CONCLUSION

The objective of the test was achieved in the relationship between stress and strain was determined as well as the yield, tensile and rapture strength, the elastic modulus, elongation and reduction of the cross- sectional area.