IMPORTANCE

The sieve analysis test is important in that it helps us determine the grading of materials that are to be used as aggregates. It ensures that particle size distribution compiles with applicable requirements and provides the data necessary to control the material of various aggregate products and mixtures containing aggregates. This data may also be useful in developing relationships concerning porosity and packing.

AIM

- To determine the particle size distribution of fine and coarse aggregates by dry sieving

APPARATUS

- Balance or scale with a minimum accuracy of 0.5g for coarse aggregate or 0.1 for fine aggregates

- Sieves

- Mechanical sieve shaker

- Oven capable of maintaining a uniform temperature of 110 ± 5 degrees

- Sample splitter to reduce the quantity of the material to the size required for sieve analysis

PROCEDURE

During the experiment, the aggregate test sample was dried to a constant weight at a temperature of 110 ± 5 degrees then cooled to room temperature. Suitable sieve sizes were then selected to furnish the information required by the specifications covering the material to be bested. Common sieves in millimeters are 37.5mm, 25mm, 19mm, 12.7mm, 9.5mm, 4.75mm, 2.36mm, 1.18mm, 0.6mm, 0.3mm, 0.15mm and 0.075mm. The sieves were placed in order of decreasing size of opening and the aggregate sample was placed on top of each sieve and agitated by hand until there were no more aggregates passing through. This process was then repeated until the aggregate sample passed through each sieve. The weight of each size increment was observed and recorded and this value was then compared to the original weight of sample placed on the sieves.

DATA COLLECTION

The table below the results obtained after the sieve analysis was done

NB: initial mass = 2044.2g

| SIEVE SIZE (mm) | MASS RETURNED (g) | CUMULATIVE MASS RETAINED | % RETURNED | % PASSING |
| --- | --- | --- | --- | --- |
| 37.5 | 0 | 0 | 0 | 100 |
| 25 | 27.5 | 27.5 | 1 | 99 |
| 19 | 199.1 | 226.6 | 11 | 89 |
| 12.7 | 1280.9 | 1507.5 | 74 | 26 |
| 9.5 | 458.1 | 1965.6 | 96 | 4 |
| 4.75 | 71.5 | 2037.1 | 100 | 0 |
| 2.36 | 2.4 | 2039.5 | 100 | 0 |

The graph below is a semi-log graph plotted as sieve size against percentage passing

DATA ANALYSIS

FINENESS MODULUS =  ∑ (cumulative percentage retained)/100

= (0 + 1 + 11 + 74 + 96 + 100 + 100)% / 100

= 3.82

DISCUSSION

The experiment was carried out successfully in that data was collected and the semi-log graph was plotted. Sieve analysis is one of the important practices in civil engineering as it is used for finding particle size distribution of particular aggregates and also used in finding the fineness modulus of aggregates. Particle distribution helps in finding the different sizes of aggregates and helps in classifying these aggregates as either fine or coarse. Fineness modulus helps in finding the maximum amount of aggregates lying in particular size of sieve of given total. This is important because the size of the aggregates has an influence on the strength of the concrete.

CONCLUSION

The aim of the experiment was achieved as the fineness modulus value calculated and the semi-log graph was plotted.

REFERENCES

Davies, J. (2004). Tensile Testing (2nd Edition ed.). ASM International.

G, J., & Barry. (2012). Mechanics of Materials (8th Edition ed.). CL Engineering.