THE UNIVERSITY OF ZAMBIA

SCHOOL OF ENGINEERING

DEPARTMENT OF CIVIL AND ENVIRONMENTALENGINEERING

CEE 3111: CIVIL ENGINEERING MATERIALS AND PRACTICES

LAB REPORT NUMBER: 1

TITLE: TENSION TEST OF STEEL AND ALUMINIUM

NAME:                     CHIWELE EMMANUEL

COMP #:                  2016134858

DUE DATE:             14-03-2019

ATTENTION:           MR. L H KAMISA

AIM

- To determine stress-strain relationship.

- To determine yield strength.

- To determine tensile strength.

- To determine elongation and reduction of cross sectional area.

- To determine modulus of elasticity.

- To determine rupture strength.

IMPORTANCE

The Tension test can be used to determine the stress-strain relation, yield strength, tension strength, modulus of elasticity of a metal and the information obtained is important for alloy development, design, quality control and comparison of different types of metals.

APPARATUS

- Universal Testing Machine.

- Vernier Callipers.

- Ruler.

SPECIMENS

- Steel(Round)

- Steel(Deformed)

- Aluminium

PROCEDURE

Two types of materials namely steel (two rods one deformed and the other round) and aluminium thus making three samples. The experiment was conducted using an Universal tensile testing machine. During the experiment, the original length of the three samples was measured. The initial diameters of the samples were also measured and also the gauge length taken midway the rods, each sample was then put in and clamped down by the upper and lower hydraulic grips. Then the tensile testing machine was then activated, calibrated and began pulling the sample down with the lower grip while the upper grip remained stationary. During this the data is stored on the display screen of the Universal tensile testing machine, the sample was pulled until it fractured and once this was observed the machine was turned off. The sample was then measured again to determine its final gauge length and diameter. The procedure was then repeated for all the specimens.

# DATA COLLECTION

Table 1: The following table shows the raw data obtained in the experiment:

|  | Aluminum | Steel (Deformed Bar) | Steel (Round Bar) |
| --- | --- | --- | --- |
| Initial Length (mm) | 530 | 534 | 534 |
| Final Length (mm | 615 | 579 | 591 |
| Initial Gage Length (mm) | 50 | 50 | 50 |
| Final Gage Length (mm) | 60 | 55 | 55 |
| Initial Diameter (mm) | 10 | 10 | 10 |
| Final Diameter (mm) | 3.10 | 7.9 | 5.3 |
| Initial Area (mm2) | 78.540 | 78.540 | 78.540 |
| Final Area (mm2) | 7.55 | 49.0 | 22.1 |

| Yield Strength (MPa) | 77.05 | 590.5 | 406.1 |
| --- | --- | --- | --- |
| Tensile Strength (MPa) | 112.671 | 683.64 | 592.2 |
| Rapture Strength (MPa) | 108.32 | 680.32 | 588.3 |
| Elasticity Modulus (GPa) | 14.5 | 62.9 | 93.7 |
| Elongation (mm) | 10 | 45 | 57 |
| %Reduction of Area | 90.4 | 37.6 | 71.9 |

# DATA ANALYSIS

STRESS-STRAIN CURVES

- ALUMINIUM:

- HIGH YIELD STEEL (DEFORMED BAR)

- MILD STEEL (ROUND BAR):

DISCUSSION

The experiment was carried out on the three samples using the Universal tensile testing Machine. This test determines important mechanical properties such as yield strength, Tensile strength, Elasticity Modulus, Rupture strength, Elongation and Reduction of Area when the materials undergo uniaxial tensile stresses.

It was observed that the steel (Deformed) recorded the largest yield of 590.5Mpa and tensile strength of 683.64Mpa and aluminium recorded the lowest Yield strength of 77.05Mpa and lowest Tensile strength of 112.671Mpa. However, Aluminium recorded the largest plastic deformation hence the largest percent reduction in area of 90.4%. The steel (Round bar) produced a yield strength of 406.1Mpa and Tensile test of 592.2Mpa.

The elastic modulus for the specimens was obtained from the slope of their respective graphs. As the results showed and its expected that steel (Deformed) would have a larger elastic modulus compared to steel (Round) but from the results obtained the mild steel specimen has the largest elastic modulus. This incoherency can be accounted to the preparation of the specimen. The grip section of the specimen should have a larger diameter compared to the gauge length so that all deformation is concentrated on the gauge length and not on the grip section.

# CONCLUSION

In conclusion, according to the results obtained after performing the experiment, the stress-strain relationship, yield strength, tensile strength and rupture strength was determined