THE UNIVERSITY OF ZAMBIA

SCHOOL OF ENGINEERING

DEPARTMENT OF ELECTRICAL AND ELECTRONIC ENGINEERING

AUTHOR             : XOSHIWE SIBANDA

COMPUTER No: 12083925

LAB TITLE         : ADDITIONAL DATA TRANSFER AND DATA MANIPULATION

LAB No           : 02

DUE DA             : 27TH APRIL, 2015

ATTENTION OF: MR. J.SHABANI

TITLE: DATA TRNSFER AND DATA MANIPULATION

- OBJECTIVES

- Use and appreciate data transfer and data manipulation.

- To deal with communication to and from the memory.

- Appreciate data representation and arithmetic instructions

- EQUIPMENT

- MAT 385 which contain an 8085A,

- 8 bit microprocessor with 40 pin dual-in-line package.

- BACKGROUND THEORY

A microprocessor accepts binary coded information (data) from its input-ports manipulates this information according to a program in its memory and outputs the results to its output-ports or stores it in memory. The instructions which constitute the program are some or all of those from the instruction-set of the particular microprocessor required to perform a particular task. The instructions of the microprocessor may be divided into five groups namely;

- data transfer

- data manipulation

- transfer of control

- input/output

- machine control

In this particular experiment we focus on Data transfer and manipulation

3.1. Data Transfer Instruction

When the data has been retrieved, the processor will process it depending on the computer program and store the results or outputs the result to an output. In both these situations, the processor needs to identify the addresses of the peripherals or memory locations involved. As a result, some addressing modes are used by the microprocessor to specify these locations and they include; direct addressing and Register Indirect addressing.

3.1.1. Direct Addressing

Using direct addressing an operand may either be read from or written to a memory location, the address of which is specified in the instruction itself. Since all the memory addresses are 16-bits, the address requires two bytes and it is for this reason that this mode is often referred to as extended addressing.  The example is;

MOV Dx, [0124H]

Meaning; MOVE             Dx ←(0124H)

STA 28F2

Meaning; STORE                   28F2 ←(A)

3.1.2. Register Indirect Addressing

Using direct addressing only the A-register may be used to store or load a value to or from memory. Thus if a value was to be stored in a memory location from say, the register B, using direct addressing, it would be necessary to transfer the contents from B to A before the store operation could be performed. A more efficient method, therefore, is to use register indirect addressing since with this mode data may be transferred between any of the processor registers and the system memory. Here, the instruction does not contain the actual memory address itself, therefore, but instead implies that the address to be used is currently stored in the HL register pair. The actual memory address is therefore obtained indirectly.

MOV A, M

Meaning; MOVING                A ←(H) (L)

MOV M,B

Meaning; MOVING               (H) (L) ←(B)

The move immediate instruction is another indirect instruction that does not directly deal with the involved address location to be interacted with.

MVI M,FF

Meaning; MOVING               (H) (L) ←(FF)

3.2. Data Manipulation I

Since microprocessor may be in a wide variety of applications, it is often necessary to represent data within the system in a number of different forms. For example, in some applications a simple unsigned binary representation is adequate but in others it may be advantageous to represent the data in a binary coded decimal form. Data in a microprocessor can be represented in signed, unsigned binary representation or Binary Coded Decimal (BCD). The user or the programmer may decide to use one of the three binary representations. The type of representation used is transparent to the microprocessor since it treats the data as an 8-bit binary pattern and it is the responsibility of the programmer to interpret this data in the form necessary to solve the problem at hand.

In this lab, some representations of data are going to be studied and therefore some arithmetic operations carried out.

Later in the experiment it will be discovered that two registers can be combined to immediate data into two consecutive memory locations, then to load register pair HL with this data and finally to store the same data into a pair of different memory locations.

3.2.1. Addition

Register addressing for the instruction of adding a content of a register into that of the accumulator.

ADD B

Meaning; ADDING                A ←(A+B)

Immediate addressing being used in adding data to the contents of the accumulator register A

ADI 0F

Meaning; ADDING               A ←(A)+0F

Indirect addressing being used in the addition instruction

ADD M

Meaning; ADDING                  A ←(A)+(H)(L)

The subtraction, increment and decrement instructions are used in like manner with specific mnemonics and environments.

3.3. Data Manipulation II

Although 8-bits are sufficient to represent a data value for many microprocessor applications, some necessitates the use of 16bits or more. The Intel 8085 provides some data transfer instructions for loading 16bit (2 bytes) immediate data into a register pair – BC, DE and HL. However, there are also instructions for incrementing and decrementing the combined 16bit contents of a register pair and also performing double length addition.

INX B

Meaning; INCREMENTING                B) (C ←BC+1

DCX D

Meaning; DECREMENTING                D) (E ←DE-1

DAD B

Meaning; ADDITION                        H) (L ←HL+(B)(C)

If more than 16-bit accuracy is required, there are no single instructions available for performing arithmetic operations and instead a number of instructions must be used. An addition of a 3 byte data would need more memory locations and so will the result of such an addition. The Intel 8085 provides additional add and subtract instructions which makes use of the carry bit.

ADC M

Meaning; ADDITION                       A ←A+HL+(CY)

SBB M

Meaning; SUBTRACTION                A ←A-HL-(CY)

3.4 DIAGRAMS -Basic Functions of the Microprocessor

BUFFER/LATCH

INSTRUCTION

REGISTER

INSTRUCTION

DECODER

CONTROL

UNIT

EXTERNAL

CONTROL

OUTPUTS

EXTERNAL

CONTROL

INPUTS

EXTERNAL DATA BUS

INTERNAL DATA

BUS

CONTROL SIGNALS TO INTERNAL REGISTERS

(a) Microprocessor control unit, instruction register, and Instruction decoder

FETCH

EXECUTE

HALT

Halt Instru.

RESET

Instruction in Microprocessor

Instruction execution complete

(a) Alternate fetch and execute cycles of a microprocessor

4.0.PROCEDURE AND DATA ANALYSIS

4.1.Experiment 1:

After the data-transfer and data manipulation exercise given in the first part of LAB2,The next exercise was to write a program that reads three 8-bit numbers from memory, adds the first two and subtracts the third one from the sum of the first two and then stores the result in the memory. The program is as reflected below. Note that that the program is in such a way that it easy to try out a lot of numbers by simply changing the contents of the registers A, B and C.

| ADDRESS | DATA | MNEMONIC | COMMENTS |
| --- | --- | --- | --- |
| 2000 | 31 | LX1 SP 20C2 | ;INITIALIZATION OF STACK-POINTER |
| 2001 | C2 |  |  |
| 2002 | 20 |  |  |
| 2003 | 06 | MVI B.DATA1 | ;(B)DATA |
| 2004 | DATA1 |  |  |
| 2005 | 0E | MVI C.DATA2 | ;(C)DATA |
| 2006 | DATA2 |  |  |
| 2007 | 3E | MVI A.DATA3 | ;(A)DATA |
| 2008 | DATA3 |  |  |
| 2009 | 80 | LX1 H 802D | ; ADDRESSING MEM |
| 200A | 2D |  |  |
| 200B | 80 |  |  |
| 200C | 70 | MOV M,B | ;(M)(B) |
| 200D | 21 | LX1 H 802E |  |
| 200E | 2E |  |  |
| 200F | 80 |  |  |
| 2010 | 71 | MOV M,C | ;(M)(C) |
| 2011 | 21 | LXI H 802F |  |
| 2012 | 2F |  |  |
| 2013 | 80 |  |  |
| 2014 | 72 | MOV M,D | ;(M)(D) |
| 2015 | 21 | LX1 H 802D |  |
| 2016 | 2D |  |  |
| 2017 | 80 |  |  |
| 2018 | 7E | MOV A,M | ;(A)(M) |
| 2019 | 21 | LX1 H 802E |  |
| 201A | 2E |  |  |
| 201B | 80 |  |  |
| 201C | 86 | ADD M | ;(A)(A+M) |
| 201D | 21 | LX1 H 802F |  |
| 201E | 2F |  |  |
| 201F | 80 |  |  |
| 2020 | 96 | SUB M | ;(A)(A-M) |
| 2021 | 21 | LX1 H 8030 |  |
| 2022 | 30 |  |  |
| 2023 | 80 |  |  |
| 2024 | 77 | MOV M,A | ;(M)(A) |
| 2025 | CF | RST | ;BACK TO MONITOR |

The program was run for several values and the results were checked in the different registers and memory locations in the assigned memory and registers specified in the program.In the case where the sum and difference was more than two separate four-bit numbers, the monitor only showed the least two significant.

  - Experiment 2:

The experiment was to write a program that adds two numbers in double precision arithmetic meaning that 16-bits are used for numerical values instead of 8-bits. The numbers were contained in register pairs BC and HL.

| ADDRESS | DATA | MNEMONIC | COMMENTS |
| --- | --- | --- | --- |
| 2000 | 31 | LX1 SP 20C2 | ;INITIALISE SP |
| 2001 | C2 |  |  |
| 2002 | 20 |  |  |
| 2003 | 01 | LX1 B 3010 | ;(B)(C)3010 |
| 2004 | 10 |  |  |
| 2005 | 30 |  |  |
| 2006 | 21 | LX1 H 0012 | ;(H)(L)0012 |
| 2007 | 12 |  |  |
| 2008 | 00 |  |  |
| 2009 | 09 | DAD B | ;DOUBLE ADDITION |
| 200A | CF | RST | ;BACK TO MONITOR |

- DISCUSSION

The laboratory experiment enhanced our knowledge of how data transfer and manipulation is achieved in a microprocessor. The data transfer and manipulation instruction in MAT385 which uses the Intel 8085 microprocessor.

Experiment  1 demonstrated that for the program to execute, the stack pointer had to be loaded with the address to the beginning of the stack memory. It was also demonstrated that a number of arithmetic could be done on the 8-bit data without the extension of the registers. As seen in the additions of some data, care had to be taken in the case were the result of the operation was more than 8-bit. Microprocessor only showed the eight least bits and the carry flags lit to show that the result was more than 8 bits.

Experiment 2 however, showed that even a 16-bit data could be manipulated when two registers were combined as BC and HL. With double precisions it was possible to load data into a register pair, hence capable of handling 16-bit data.

The addressing modes were also demonstrated they were the algorithms used to locate the memory in which the data to be manipulated had being stored. This lab has showered with a written program in hand, an 8085A microprocessor can be used to process information and hence control real life processes. Though microprocessors are general purpose electronic processing devices which can be programmed to execute a number of tasks, they also have limitations such as the memory size (about 64k), does not support floating point operation and that they can easily crash.

- CONCLUSION

The objectives of the experiment being Data transfer and manipulation were achieved simply by writing of programs, debugging them and therefore having hands on writing programs in assembly language. The relation between the assembly language and the machine code was also seen.Familiarisation of the microprocessor instructions (MOV, MVI, ADD and so on) was also done during the experiment. Two of the addressing modes; the direct and indirect addressing, were used to locate and transfer data from/to the memory.

- APPLICATION

### Microprocessors find its application mostly  in electronic devices where data manipulation is required at a medium scale. This includes ; instrumentation

Office Automation and Publication,control and it has found being used in;

- Calculators

- Accounting system

- Games machine

- Complex Industrial Controllers

- Traffic light Control

- Data acquisition systems

- Multi user, multi-function environments

- Military applications

- Communication systems

- REFERENCES

- MAT 385 operating manuals (Exercises 1 to 3)

- EE 481 lab manual – UNZA, 2011.

- www.intel.com/microprocessor/8085A.