TITLE: ADDITIONAL DATA TRNSFER AND DATA MANIPULATION I & II

- OBJECTIVES

- Use and appreciate data transfer and data manipulation.

- To deal with communication to and from the memory.

- Appreciate data representation and arithmetic instructions

- EQUIPMENT

- MAT 385 which contain an 8085A,

- 8 bit microprocessor with 40 pin dual-in-line package.

- BACKGROUND THEORY

A microprocessor accepts binary coded information (data) from its input-ports manipulates this information according to a program in its memory and outputs the results to its output-ports or stores it in memory. The instructions which constitute the program are some or all of those from the instruction-set of the particular microprocessor required to perform a particular task. The instructions of the microprocessor may be divided into five groups namely;

- data transfer

- data manipulation

- transfer of control

- input/output

- machine control

In this particular experiment we focus on Data transfer and manipulation

3.1. Data Transfer Instruction

When the data has been retrieved, the processor will process it depending on the computer program and store the results or outputs the result to an output. In both these situations, the processor needs to identify the addresses of the peripherals or memory locations involved. As a result, some addressing modes are used by the microprocessor to specify these locations and they include; direct addressing and Register Indirect addressing.

3.1.1. Direct Addressing

Using direct addressing an operand may either be read from or written to a memory location, the address of which is specified in the instruction itself. Since all the memory addresses are 16-bits, the address requires two bytes and it is for this reason that this mode is often referred to as extended addressing.  The example is;

MOV Dx, [0124H]

Meaning; MOVE             Dx ←(0124H)

STA 28F2

Meaning; STORE                   28F2 ←(A)

3.1.2. Register Indirect Addressing

Using direct addressing only the A-register may be used to store or load a value to or from memory. Thus if a value was to be stored in a memory location from say, the register B, using direct addressing, it would be necessary to transfer the contents from B to A before the store operation could be performed. A more efficient method, therefore, is to use register indirect addressing since with this mode data may be transferred between any of the processor registers and the system memory. Here, the instruction does not contain the actual memory address itself, therefore, but instead implies that the address to be used is currently stored in the HL register pair. The actual memory address is therefore obtained indirectly.

MOV A, M

Meaning; MOVING                A ←(H) (L)

MOV M,B

Meaning; MOVING               (H) (L) ←(B)

The move immediate instruction is another indirect instruction that does not directly deal with the involved address location to be interacted with.

MVI M,FF

Meaning; MOVING               (H) (L) ←(FF)

3.2. Data Manipulation I

Since microprocessor may be in a wide variety of applications, it is often necessary to represent data within the system in a number of different forms. For example, in some applications a simple unsigned binary representation is adequate but in others it may be advantageous to represent the data in a binary coded decimal form. Data in a microprocessor can be represented in signed, unsigned binary representation or Binary Coded Decimal (BCD). The user or the programmer may decide to use one of the three binary representations. The type of representation used is transparent to the microprocessor since it treats the data as an 8-bit binary pattern and it is the responsibility of the programmer to interpret this data in the form necessary to solve the problem at hand.

In this lab, some representations of data are going to be studied and therefore some arithmetic operations carried out.

Later in the experiment it will be discovered that two registers can be combined to immediate data into two consecutive memory locations, then to load register pair HL with this data and finally to store the same data into a pair of different memory locations.

3.2.1. Addition

Register addressing for the instruction of adding a content of a register into that of the accumulator.

ADD B

Meaning; ADDING                A ←(A+B)

Immediate addressing being used in adding data to the contents of the accumulator register A

ADI 0F

Meaning; ADDING               A ←(A)+0F

Indirect addressing being used in the addition instruction

ADD M

Meaning; ADDING                  A ←(A)+(H)(L)

The subtraction, increment and decrement instructions are used in like manner with specific mnemonics and environments.

3.3. Data Manipulation II

Although 8-bits are sufficient to represent a data value for many microprocessor applications, some necessitates the use of 16bits or more. The Intel 8085 provides some data transfer instructions for loading 16bit (2 bytes) immediate data into a register pair – BC, DE and HL. However, there are also instructions for incrementing and decrementing the combined 16bit contents of a register pair and also performing double length addition.

INX B

Meaning; INCREMENTING                B) (C ←BC+1

DCX D

Meaning; DECREMENTING                D) (E ←DE-1

DAD B

Meaning; ADDITION                        H) (L ←HL+(B)(C)

If more than 16-bit accuracy is required, there are no single instructions available for performing arithmetic operations and instead a number of instructions must be used. An addition of a 3 byte data would need more memory locations and so will the result of such an addition. The Intel 8085 provides additional add and subtract instructions which makes use of the carry bit.

ADC M

Meaning; ADDITION                       A ←A+HL+(CY)

SBB M

Meaning; SUBTRACTION                A ←A-HL-(CY)

- PROCEDURE AND RESULTS

Exercise 4.1:

The program was listed and coded as shown in the table below using a combination of immediate and direct addressing first to store immediate data into two consecutive memory locations, then to load register pair HL with this data and finally to store the same data into a pair of different memory locations. The program was executed by using a single step command and the contents of the of appropriate processor register or memory location examined.

| ADDRESS | DATA | MNEMONIC | COMMENTS |
| --- | --- | --- | --- |
| 2800 | 3E | MVI A, FF | ;(A) (FF) |
| 2801 | FF |  |  |
| 2802 | 32 | STA 28A2 | ;(28A2) (A) |
| 2803 | A2 |  |  |
| 2804 | 28 | MVI A, EE | ;(A) EE(hex) |
| 2805 | 3E |  |  |
| 2806 | EE |  |  |
| 2807 | 32 | MVI A.DATA3 | ;(A)DATA |
| 2808 | A3 |  |  |
| 2809 | 28 | LX1 H 802D | ; ADDRESSING MEM |
| 280A | 2A |  |  |
| 280B | A2 |  |  |
| 280C | 28 | MOV M,B | ;(M)(B) |
| 280D | 22 | LX1 H 802E |  |
| 200E | A4 | SHLD 28A4 | ;(28A4) (L) |
| 200F | 28 |  | ;(28A5) (H) |
| 2010 | A4 |  |  |

Exercise 4.2:

The program was listed and coded as shown in the table below using a combination of immediate and register indirect addressing .A memory address was first loaded into register H and L using immediate addressing and then a value was loaded into this memory location using register indirect addressing. Finally, the value was loaded into two further registers again using register indirect addressing. The program was executed by using a single step command and the contents of the of appropriate processor register or memory location examined.

| ADDRESS | DATA | MNEMONIC | COMMENTS |
| --- | --- | --- | --- |
| 2800 | 21 | LX1 H, 28A | ;(H) 28(hex) |
| 2801 | A0 |  | ;(L) A0(hex) |
| 2802 | 28 |  |  |
| 2803 | 36 | MVI M ,AA | (28A0) AA(hex) |
| 2804 | AA |  |  |
| 2805 | 46 | MOV B, M | (B )AA |
| 2806 | 4E | MOV C ,M | (C) AA |

Exercise 4.3

The following Assembly language was listed and coded to answer to exercise 4.3.The program was executed and the contents of appropriate register or memory location examined to verify correct execution.

| ADDRESS | DATA | MNEMONIC | COMMENTS |
| --- | --- | --- | --- |
| 2800 | 3E | MVI A,FF | (A) (FF) |
| 2801 | FF |  |  |
| 2802 | 32 | STA 28FF | 28FF A |
| 2803 | FF |  |  |
| 2804 | 28 |  |  |
| 2805 | 2A | LHLD 28FF | (L) FF |
| 2806 | FF |  | (H) 28 |
| 2807 | 28 |  |  |
| 2808 | 06 | MOV B,M |  |
| 2809 | M | LDCD B |  |

Exercise 5.2

The following program was loaded into memory and using a single step command the correct execution of the program was verified. This program uses unsigned binary number representation. The two registers A and B are first loaded with immediate data and their contents are added together. A third number is then subtracted from the contents of A using immediate addressing and finally the new contents of A are decremented by unity.

| ADDRESS | DATA | MNEMONIC | COMMENTS |
| --- | --- | --- | --- |
| 2800 | 3E | MVI A, 53 | ;(A) 53(hex) |
| 2801 | 53 |  |  |
| 2802 | 06 | MVI B, 3A | ;(B) 3A(hex) |
| 2803 | 3A |  |  |
| 2804 | 80 | ADD B | (A) (A )+ (B)(hex) |
| 2805 | DE | SBI 8C | (A) (A )- (8C)(hex) |
| 2806 | BC |  |  |
| 2807 | 3D | DRC A | A) (A )- (1) |
| 2808 | A |  |  |

Exercise 5.3 The following program was loaded into memory and using a single step command the correct execution of the program was verified. The following program uses two’s complement signed representation to produce a summation producing a negative result.

| ADDRESS | DATA | MNEMONIC | COMMENTS |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 2800 | 3E | MVI A, 23 | ;(A) 23(hex) |  |  |  |
| 2801 | 23 |  |  |  |  |  |
| 2802 | 06 | MVI B, B | ;(B) BB(hex) |  |  |  |
| 2803 | B8 |  |  |  |  |  |
| 2804 | 80 | ADD B | (A) (A )+ (B)(hex) |  |  |  |
| 2805 | DE | SBI DB | (A) (A )- (DB)(hex) |  |  |  |
| 2806 | DB |  |  |  |  |  |
| 2807 | 3D | DRC A | A) (A )- (1) |  |  |  |
| 2808 | A |  |  |  |  |  |

Exercise 6.1

From the provided Assembly language code, the following codding form was written to load the program into memory and run it. This code emphasized multi precision Arithmetic.

| ADDRESS | DATA | MNEMONIC | COMMENTS |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2800 | 26 | MVI H, 28 | ;(HL) 28F3(hex) |  |  |  |  |
| 2801 | 28 |  |  |  |  |  |  |
| 2802 | 2E | MVI L, F3 |  |  |  |  |  |
| 2803 | F3 |  |  |  |  |  |  |
| 2804 | 3A | LDA 28F0 | Adds 1st pair of bytes |  |  |  |  |
| 2805 | 28 | ADD M |  |  |  |  |  |
| 2806 | F0 | STA 28F0 |  |  |  |  |  |
| 2807 | 86 |  |  |  |  |  |  |
| 2808 | 32 | LDA 28F1 |  |  |  |  |  |
| 2809 | 28 | ADC M | Adds 2nd pair of bytes |  |  |  |  |
| 280A | FO | STA 28F1 |  |  |  |  |  |
| 280B | 23 | INX H | Increment HL |  |  |  |  |
| 280C | 3A | LDA 28F1 |  |  |  |  |  |
| 280D | 28 | ADC M | Adds 2nd pair of bytes |  |  |  |  |
| 280E | F1 | STA 28F1 |  |  |  |  |  |
| 280F | 8E |  |  |  |  |  |  |
| 2816 | 32 |  |  |  |  |  |  |
| 2817 | 28 |  |  |  |  |  |  |
| 2818 | F1 |  |  |  |  |  |  |
| 2819 | 23 | INX H |  |  |  |  |  |
| 2820 | 3A | LDA 28F2 |  |  |  |  |  |
| 2821 | 28 | ADC M | Adds 3rd pair of bytes |  |  |  |  |
| 2822 | F2 | STA 28F2 |  |  |  |  |  |
| 2823 | 8E |  |  |  |  |  |  |
| 2824 | 32 |  |  |  |  |  |  |
| 2825 | F2 |  |  |  |  |  |  |

Exercise 6.2

From the Binary coded decimal arithmetic, the following program was written to perform BCD arithmetic. The program was loaded into memory and run.

| ADDRESS | DATA | MNEMONIC | COMMENTS |
| --- | --- | --- | --- |
| 2800 | 26 | MVI H, 28 | ;(HL) 28F2(hex) |
| 2801 | 28 |  |  |
| 2802 | 2E | MVI L, F2 |  |
| 2803 | F2 |  |  |
| 2804 | 3A | LDA 28F0 | Adds 1st pair of bytes |
| 2805 | F0 | ADD M | And form corrected |
| 2806 | 28 | DAA | sum |
| 2807 | 86 | STA 28F0 |  |
| 2808 | 27 |  |  |
| 2809 | 32 |  |  |
| 280A | FO |  |  |
| 280B | 28 |  |  |
| 280C | 23 | INX H | Increment HL |
| 280D | 3A | LDA 28F1 | Adds 2nd pair of bytes |
| 280E | F1 | ADC M | And form corrected |
| 280F | 28 | DAA | sum |
| 2816 | 8E |  |  |
| 2817 | 27 |  |  |
| 2818 | 32 | STA 28F1 |  |
| 2819 | F1 |  |  |
| 2820 | 28 |  |  |

Exercise 6.3

The following program loads immediate data into registers A and B and then performs a series of logical operations.

| ADDRESS | DATA | MNEMONIC | COMMENTS |
| --- | --- | --- | --- |
| 2800 | 3E | MVI A, F0 | ;(A) F0(hex) |
| 2801 | F0 |  |  |
| 2802 | 06 | MVI B, 0F | ;(B) OF(hex) |
| 2803 | 0F |  |  |
| 2804 | B8 | CMP B | ;Z 0, ;CY O |
| 2805 | 07 | RCL | ;(A) E1, ;CY 1 |
| 2806 | 40 | ANI 81 | ;(A) 81, ;CY O |
| 2807 | 81 |  |  |
| 2808 | B0 | ORA B | ;(A) 8F, CY O |

- DISCUSSION

The laboratory experiment enhanced our knowledge of how data transfer and manipulation is achieved in a microprocessor. The data transfer and manipulation instruction in MAT385 which uses the Intel 8085 microprocessor was investigated and specific functional concepts analysed with specific assignment.

- Data Transfer

In exercise 4.1, the concepts of immediate and direct addressing were used first to store immediate data into two consecutive memory locations, then to load register pair HL with this data and finally to store the same data into a pair of different memory locations.

In exercise 4.2, the additional concept of register indirect addressing was used to load data into memory locations H and L where they were then loaded into registers.

Exercise 4.3, was then done using a combination of direct addressing and indirect addressing to move around data in registers and memory locations.

We established that using the direct addressing, the 16-bit memory address is specified in the instruction itself while in the indirect addressing the memory address is implied as the current content of the register pairs H and L.

b) Data Manipulation I

This part of the laboratory experiment introduced three forms of number representation available to the programmer. Unsigned binary number representation and the two’s compliment number representation were investigated in exercise 5.2 and 5.3. Some basic arithmetic instructions were examined together with the meaning of the various status bits which make up the flag register. With the knowledge of data representation, multi-precision arithmetic’s were carried out.

- Data Manipulation II

BCD arithmetic’s and logical operations were performed in exercise 6.2 and 6.3. This portion of the laboratory experiment introduced additional addition and subtraction with carry and the BCD decimal adjust. Familiarisation of the microprocessor instructions (MOV, MVI, ADD and so on) was also done during the experiment. Two of the addressing modes; the direct and indirect addressing, were used to locate and transfer data from/to the memory.

- CONCLUSION

The objectives of the experiment which were; Data transfer and manipulation were achieved and the relation between the assembly language and the machine code was also  established .

- REFERENCES

EEE 3132 Laboratory manual – UNZA, 2015

THE UNIVERSITY OF ZAMBIA

SCHOOL OF ENGINEERING

DEPARTMENT OF ELECTRICAL AND ELECTRONIC ENGINEERING

AUTHOR             : XOSHIWE SIBANDA

COMPUTER No: 12083925

LAB TITLE         : ADDITIONAL DATA TRANSFER AND DATA MANIPULATION

LAB No           : 02

DUE DA             : 27TH APRIL, 2015

ATTENTION OF: MR. J.SHABANI