UNIVERSITY OF NAMIBIA EXAMINATIONS

EXAMINATION QUESTION PAPER

DECEMBER SUPPLEMENTARY EXAMINATIONS 2013

| FACULTY | SCIENCE |
| --- | --- |
| DEPARTMENT | GEOLOGY |
| SUBJECT | GLY 3712- STRUCTURAL GEOLOGY I -THEORY |
| SUBJECT CODE | GLY 3712-PAPER 1 |
| DATE | OCTOBER 2017 |
| DURATION | 3 HOURS |

EXAMINER: DR. B.S. MAPANI

MODERATOR: PROF. A.F. KAMONA

INSTRUCTIONS

- CLOSED BOOK EXAMINATION

- READ QUESTIONS CAREFULLY

- ANSWER FIVE (5) QUESTIONS ONLY

- START EACH QUESTION ON A SEPARATE PAGE

- (a) Discuss how strain varies with (i) temperature (ii) confining pressure (iii) fluid pressure, in a rock undergoing deformation at shallow crustal levels, i.e., 1-4 km and at deeper crustal levels, i.e., 6-12 km. 						[15]

(b). An initial angle of 90 is homogeneously distorted until it is (i) 84, 60, 45 and 30.  What is the strain rate if the time taken to accomplish these strains are 10 seconds, 100 seconds, 1000 seconds, 10000 seconds respectively.  Draw a graph of strain versus strain rate and comment on its significant, and thus on the type of deformation that has taken place.									[10]

- The Heroes’ acre in Windhoek is characterized by dolerite dimension stones at the bottom resting on quartzite rock formation.  Above the dolerite is a beautiful marble obelisk that towers over Windhoek.  The obelisk has a cross-section that can be approximated to a hexagon, and measures 5 metres tall, with sides of 2 m each for the six sides.  The obelisk rests a slab of dolerite, that measures 5 m x5 m and 2 metres in height.

- Given that the density of marble is 2 g/cm3, and that of dolerite is 3.5g/cm3 and that o quartzite is 2.5g/cm3, calculate the force that is exerted on the basal underlying quartzites. 						[5]

- What is the stress on the quartzite layers on which the foundation of the heroes’ acre is located?							[10]

- if the quartzites are dipping at an angle of 30 to the north, calculate the effective force being transmitted into the ground. 			[10]

- The Zintuzikazanda Nikuzionela mylonite has been tested in a laboratory and found to be quite strong.  A differential stress of 700MPa is required to fracture it when 1 is 800 MPa.  The coefficient of friction at failure is 0.77777.

- calculate the normal stress in the Bamba Zonke mylonites that induces failure.										[10]

- What is the shear stress, at failure in the mylonites? 		[10]

- calculate the cohesiveness of the mylonites and comment on its competence.  										[5]

4.	(a)	A poly-deformed area has recorded three deformation episodes, D1-D4.  D2 is characterised by refolded folds, whose F2 axes plunge to the NE at 30 and F1 fold axes plunge to NW and SE at 40 and 30 respectively.  D3 is a crenulation-cleavage forming event, with hinge lines of the microfolds being parallel to the F1 folds. D4 is a major faulting event.

(i) If down dip lineations (L1) had developed during the development of F1, what is the orientation of L1 after the F2 episode? 	(10)

(ii) What was the nature of D1?  	(5)

(iii) What is the orientation of the crenulation lineations?	 (5)

(iv) Deduce the orientation of the principal compressive stresses 1, 2, and 3 during D2 and D3. (5)	[25]

5	(a) Compare and contrast Grain boundary Diffusion from Dislocation Creep. 	[10]

(b)Discuss the significance of decompression textures and compare them to replacement textures, using sketches and relevant mineral equilibria. Which deformation mechanism would be responsible for the formation of each of these textures? 	[15]

END OF EXAM