MOBILITY OF CHEMICAL ELEMENTS

General

Mobility is the ability of a chemical element to migrate from one part of the earth to the other. The circulation of a chemical element in the earth is described by a geochemical cycle. The migration of an element may take place during geological, ecological, pedological  or technological time. The geochemical cycle has two main parts the geological part (major cycle or primary geochemical environment) and the landscape part(minor cycle, secondary/surficial geochemical environment). The different parts of the geochemical cycle are given in figure 1.

The establishment of factors that control mobility of chemical elements in the geochemical environment is one of the major goals of geochemistry. Element mobility is an important concept in high temperature geochemistry, environmental geochemistry and exploration geochemistry.

Mobility of elements in the surficial environment

The secondary geochemical environment is characterized by low temperatures, low pressure, high abundance of fluids and high flow rates of transport agents  (gases, liquids). The conditions that exist in the secondary environment favor higher mobility of chemical elements in this environment compared to the primary environment. The elements may migrate in solid state or dissolved state. The major factors that control aqueous mobility of chemical elements in the secondary environment include the following: pH ( acidity of the environment), Eh (oxidation potential), ionic potential of the element, stability of the minerals in which an element occurs, presence of specific complexing agents in the environment, nature and abundance of adsorbents in the environment. The rate of migration of any given element in the surficial environment is drastically reduced by the presence of geochemical barriers. The geochemical barriers are lithologic boundaries in the supergene zone, where conditions of migration change drastically and concentrations of chemical elements begin to rise. On the basis of their geometrical properties Perel’man identified two kinds of geochemical barriers: (1) areal barriers which may extend over large areas of country (e.g. grasslands, bogs, forests) and (2) linear barriers that cover small areas ( slope discontinuities, fault zones, spring lines). There are three main types of  geochemical barriers: (1) mechanical barriers, (2) physicochemical barrier and (3) biological barrier. The size of geochemical barriers may vary from a few cm in soils to several kilometers in some geological formations.

Mechanical barriers develop where the velocity of the landscape geochemical flow of water or air changes abruptly. Typical example is the development of  a placer deposit of tin, gold or diamond due to an abrupt change in the slope of the stream bed. This change in the slope results in the selective separation of heavy mineral grains from the stream water. Mechanicla barriers may also exist in aquifers due to changes in texture and permeability characteristics of rocks.

Physicochemical barriers include the following types:

- The oxygen type for Fe, Mn, Co and S.

- The hydrogen sulfide (reducing type) type for V, Fe, Cu, Co, As, Se, Ag, Ni, Zn, Cd, Hg, Pb and U.

- The sulfatic and carbonatic type for Ca, Ba and  Sr,

- The alkaline type for Ca, Mg, Sr, V, Cr, Mn, Fe, Co, Ni, Cu, Zn, Cd and Pb.

- The acid type for SiO2.

- The evaporate type for Li, N, F, Na, Mg, S, Cl, K, Ca, Zn, Sr, Rb, Mo, I and U.

- The sorptive type for Mg, P, S, K, Ca, V, Cr, Co, Ni, Cu, Rb, Mo, Zn, As, Hg, Pb, Ra and U.

Biological barriers include humus layers in forests, living organisms which accumulate carbon and other elements such as N, P and S; and peat bog.

Acidity of the environment/pH

The pH of the aqeous environment has a major impact on the mobility of a large number of chemical elements. The precipitation of a large number of oxides and hydroxides of elements from dilute aqueous solutions is controlled by pH. The control of pH on the precipitation of hydroxides is given below:

pH		Hydroxide  precipitated		Natural media

11

Magnesium				Alkalic soils

10

9

Mn2+					Seawater

8

7		Zinc					Riverwater

6		Copper					Rainwater

5		Bivalent iron

4		Aluminium				Peat water

3							Mine waters

Trivalent iron

Acid thermal springs

1

The pH of terrestrial waters is controlled by the buffer system CaCO3-CO2-H2O. A saturated solution of CO2 in the atmosphere has a pH of 5.7. A saturated solution of calcite in air has an pH of 8.4.

Examples

Fe 		The solubility product of Ferric hydroxide is given by the equation.

K = [Fe3+ ] [OH-] 3

[Fe3+] = K/ [OH-] 3 , but [OH] = Kw/[H+]

[Fe3+] = K[H+] 3/Kw3

But K= 10-38.6 and Kw = 10-14.2

At 18 0C K/Kw = 104

At pH = 7,  [Fe3+] = 10-17 mol/l, at pH = 6; [Fe3+] = 10-14 mol/l

The total amount of dissolved iron at different pH is:

pH		Amount

  - 3 x 10-11 mg /l

- 4 x 10-10 mg/l

7		4 x 10-8   mg/l

6		5 x 10-6    mg/l

Thus the content of dissolved Fe in seawater is 0.008 ppm while in river water it is about 1 ppm.

Al and Si

The solubility of Al(OH)3  and SiO2 is a function of pH. In natural environments Si may be mobilized readily under alkaline conditions. Al is not mobile in the pH range of 4 to 9. Al is mobile under strong acidic and alkaline conditions.

Other metals

Metals are highly soluble in acid solutions and tend to form insoluble hydroxides with an increase in pH. Consider the solubility of Cu as  a function of  pH.

Cu2+ (aq) + H2O (l) = CuO (s) + 2H+ (aq)

KCuO  =  [H+]2/[Cu2+] = 10-7.35

Log  [Cu2+] =2 log [H+] + 7.35

Log  [Cu2+] = 7.35 – 2pH

At pH = 7 copper concentration  is 14 ppb.

Redox potential

The redox potential ( Eh ) may affect the mobility of elements that may exist in more than one oxidation state. The following elements may exist in the listed oxidation states: Fe (0, 2, 3); Mn (2, 3, 4); S (-2, 0, 6); V ( 3, 4, 5); Cu ( 0, 1, 2); N (-3, 0, 5); U (4, 6); Cr (3, 6); V (3, 4, 5); C (0, -4, +4); N (-3, 0, 5). The stability of an element in a particular oxidation state depends on the energy change (Eh) involved in adding or removing electrons. This redox potential for any given reaction is measured relative to the reference reaction given below.

H2 = 2H+ + 2e   (1 atm. And 250C)

Redox potentials (in volts) for selected reactions are given below:

CO2+ = CO3+ + e			1.84

Ni2+ + 2H2O = NiO2 + 4H+ + 2e	1.75

Mn2+  = Mn3+ + e			1.51

Pb2+ + 2H2O = PbO2 + 4H+ + 2e	1.46

2H2O = O2 + 4H+ + 4e			1.23

NH4  + 3H2O = NO3- + 10 H+ + 8e	0.84

Fe2+ = Fe3+ + e 			0.77

S-2 + 4 H2O = SO42- + 8H+ + 8e	0.14

H2 = 2H+ + 2e				0.0

So H2 is able to reduce Fe3+. Fe3+ reduces Mn3+

The oxidation potential varies with varying concentration of the reacting substances.  A variation in pH produces large changes in oxidation potentials  for reactions that involve H+ and OH- ions. The range of  oxidation potentials of natural environments determines the reactions that may take place. Chemical reactions in aqueous media are theoretically limited to those with oxidation potentials between those for the reactions.

2H2O = O2 + 4H+ + 4e		E0 = 1.23  volts

2H+ + 2e = H2			E0 = 0.0 volts

The oxidation potential of natural environments at pH 7 may lie between 0.81 and -0.41 volts. The redox potential plays a major role in the separation of some element associations during supergene processes. Fe-Ni-Co association in mafic rocks is destroyed in the surficial environment due to differences in the oxidation potentials of Fe2+, Co2+ and Ni2+. Fe is also separated from Mn.

Eh and pH conditions in the secondary environment

The range of Eh and pH of natural environments is very important in the study of mobility of chemical elements and stability of different substances in the surficial environments. The Eh and pH conditions of natural waters are given in figure. These conditions are oxidizing environment (mine waters, rainwater, stream water, normal ocean water),  transitional environment (bog waters, groundwaters) and Reducing environment ( water-logged soils, euxenic marinewaters, organic –rich saline waters). The controls on Eh and pH of different waters are given in the table below (table 1). The effect of ph and Eh on the mobility of elements in surficial environments is summarized in table 2.

Quantitative measure of mobility

Perel’man defined a parameter known as Coefficient for aqueous migration (K) which can be used to estimate mobilities of elements.

K = 100M/(a.N)

Where M is the conc. Of the element in the drainage water (in mg/l), a is the total mineral residue in the water (%), and N is the concentration of the element in the rocks (%).

Ionic Potential

The surface charge density on a dissolved ion determines  the extent to which an ion interacts with the water molecules in the secondary environment. The dipole nature of the water molecule allows it to be attracted to both cations and anions. The number of water molecules that can be packed around the ion depends on the ionic potential (Z/r). The larger  Cs+ (0.60) ion is less hydrated than the smaller Li+ (1.5).

The ionic potential of an element affects its mobility in the secondary environment. Elements with low ionic potential (Na, Ca, Mg) remain in the solution. Elements with intermediate ionic potentials are precipitated by hydrolysis and elements with higher ionic potentials form very soluble complex oxy-anions (N, S, P, C, B).

Table 1  Range and controls of Eh and pH of various natural materials

| Material | pH range | Eh range (mV) | Controls |
| --- | --- | --- | --- |
| Meteoric waters | 4-8 | +800 to +300 | Amount of dissolved CO2, O2. |
| Peat bogs Depressions | 7-8 | +500 to negative values | Topograpphy, organic reactions |
| Mounds | 3-7 | “ |  |
| Soils | 2.8-10 + | 750 t0 - 350 | Amount of water and kinds of minerals present; organic content |
| Groundwater | 5-9 | +500 to - 100 | Rock environment, organic reactions |
| Mine water Oxidized zone Primary zone | 2-9 6-9 | +800 to +200 +200 to -100 | Oxidation of pyrite Rock environment |
| Freshwater rivers and lakes | 4 - 10 | +600 to -100 | Amount and kind of dissolved material |
| Freshwater sediments | 4-9 | +600 to -200 | Sediment composition |
| Marginal marine sediments (deltas etc) | 5-10 | +500 to - 400 | Degree of isolation; Kind and amount of flora and fauna; sediment composition |
| Seawater | 6-10 | +500 to -200 | Algae; balance between dissolved CO2 and CaCO3 |
| Open-sea sediments | 6-9 | +600 to -400 | Algae; balance between dissolved CO2 and CaCO3; sediment composition |
| Evaporites | 6-10 | +600 to -500 | Composition of evaporate water; organic content |
| Geothermal waters | 0.85-9.5 | +700 to – 185 | Numerous factors |
| Connate waters | 5-8 | +100 to -300 | Isolation from air; rock environment |

Source: Geochemistry , Brownlow, !979.

Solubility of Minerals The solubility of the mineral in which an element occurs may affect the mobility of a given element. Fe and Al are highly immobile due to the high stability of their oxides in the secondary environment. Pb in PbS is highly immobile due to the formation of a thin white film of anglesite (PbSO4) during chemical weathering. Zn and Cu in sulphate form are highly mobile. The mobility of Si is low because it occurs in silicates that are highly insoluble. Na is mobile in NaCl but its mobility is low in silicate minerals.

Table 2 Mobility of elements in surficial environments

| Relative mobility | Oxidising (pH 5- 8) | Oxidising (pH < 4) | Reducing |
| --- | --- | --- | --- |
| Highly mobile (K>10) | Cl, Br, I, S, Rn, He, C, N, Mo, B (se, Te, Re ?) | Cl, Br, I, S, Rn, He, C, N, B | Cl, Br, I, Rn, He |
| Moderately mobile (K= 1-10) | Ca, Na, Mg, Li, F, Zn, Ag, U, V, As (Sr, Hg, Sb ?) | Ca, Na, Mg, Sr, Li, F, Zn, Cd, Hg, Cu, Ag, Co, Ni, U, V, As, Mn, P | Ca, Na, Mg, Li, Sr, Ba, Ra, F, Mn |
| Slightly mobile (K= 0.1-1) | K, Rb, Ba, Mn, Si, Ge, P, Pb, Cu, Ni, Co (Cd, Be, Ra, In, W?) | K, Rb, Ba, Si, Ge, Ra | K, Rb, Si, P, Fe |
| Immobile (K < 0.1) | Fe, Al, Ga, Sc, Ti, Zr, Hf, Th, Pa, Sn, Rare earths, Pt metals, Au (Cr, Nb, Ta, Bi, Cs?) | Fe, Al, Ga, Sc, Ti, Zr, Hf, TH, Pa, Sn, Rare earths, Pt metals, Au, As, MO, Se | Fe, Al, Ga, Ti, Zr, Hf, Th, Pa, Sn, Rare earths, Pt, metals, Au, Cu, Ag, Pb, Zn, Cd, Hg, Ni, Co, As, Sb, Bi, U, V, Se, Te, Mo, In, Cr (Nb, Ta, Cs?) |

Adsorption and Ion exchange

Adsorption is the binding of dissolved substances onto the surfaces of solids or colloidal particles. Two types of adsorption are : (1) physical (vanderwaals) and (2) chemical adsorption.

Physical adsorption is characterized by a loose bonding of the adsorbate to the adsorbent. Chemical adsorption is characterized by a firm chemical bonding of the adsorbate. Principles that govern the adsorption process include the following:

- The degree of adsorption increases with increase in the surface area of the adsorbent.

- Adsorption is favored if  the adsorbate forms a compound of low solubility with the adsorbent ( e.g. adsorption of PO43- by Fe(OH)3.

- The amount of a substance adsorbed from solution increases with its concentration in that solution.

- Highly charged ions are adsorbed more readily than lower charged ions.

Through adsorption processes many ions may be removed from solution or natural waters. The adsorptive capacity of clay minerals is relatively high. Many complex ions ( MoO, AsO) and ions of heavy elements ( Cu, Pb, Zn, Cd, Co) are adsorbed from aqueous solutions by adsorbents such as clay, organic matter, amorphous silica,  aluminium hydroxide, ferric hydroxide, hydrated manganese dioxide and carbonate minerals.

The adsorptive power of hydrated manganese dioxide is great and results in increased concentrations of Li, K, Ba, B, Ti, Co, Ni, Cu, Mo, As V, Zn, Pb  and W in Mn ores.

Ca-clay  + Zn2+ = Zn-clay + Ca2+

Or	AX + B+   =  BX + A+

Adsorbents in soils act as temporary repositories of ions released by chemical weathering or addition of fertilizers. Adsorbents that sink in the oceans scavenge metal ions from seawater and transfer them to the bottom of the oceans. Deep-sea sediment is enriched in many metals. The charge on colloidal particles may depend on the pH of the environment. Substances that form colloidal particles in natural environments include clay minerals, quartz, amorphous silica, hydrated manganese oxides,  albite,  hematite, magnetite,  goethite, limonite (Fe2O3.nH2O), anatase, gibbsite, corundum and periclase.

The capacity of a substance to act as an ion exchanger varies widely. The CEC of clays is given below:

Clay mineral		Cation exchange capacity

Kaolinite			3-15

Chlorite			10-40

Illite				10-40

Allophane			70

Smectite			70-100

Vermicullite			100-150

The charge on an adsorbent may be positive or negative.

The charges on some adsorbents are given below.

Positively charged adsorbents

Al(OH)3

Ferric Hydroxide

Chromic hydroxide

Titanium dioxidehydrate

Zirconium dioxidehydrate

Negatively charged adsorbents

Amorphous silica

Quartz

Ferric hydroxide

Vanadium pentoxide hydrate

Mn dioxide hydrated

Humus colloids

Sulfide sols

Kaolinite, montmorillonite

Negatively charged particles adsorb cations  while positively charged particles adsorb anions.

The adsorption coefficient of adsorbents for adsorbates is not uniform. There is a tendency for certain ions to be strongly adsorbed than for others. The extent to which ions are adsorbed on different materials are given below:

Table:  Relative affinity for adsorption of some cations

|  | Mn-oxides | Amorphous Fe-oxides | Goethite | Amorphous Al-oxides | Humic substances |
| --- | --- | --- | --- | --- | --- |
| Greatest | Cu2+ | Pb2+ | Cu2+ | Cu2+ | Cu2+ |
|  | Co2+ | Cu2+ | Pb2+ | Pb2+ | Ni2+ |
|  | Mn2+ | Zn2+ | Zn2+ | Zn2+ | Co2+ |
|  | Zn2+ | Ni2+ | Co2+ | Ni2+ | Pb2+ |
|  | Ni2+ | Cd2+ | Cd2+ | Co2+ | Ca2+ |
|  | Ba2+ | Co2+ |  | Cd2+ | Zn2+ |
|  | Sr2+ | Sr2+ |  | Mg2+ | Mn2+ |
|  | Ca2+ | Mg2+ |  | Sr2+ | Mg2+ |
| Least | Mg2+ |  |  |  |  |

Complex Formation

Dissolved ions may combine with other dissolved components to form strong complexes that may increase the proportion of these ions in dissolved state. The formation of complexes increases the mobility of the elements involved. Sholkovitz  (1983) observed that Pu4+ forms strong  complexes with dissolved organic compounds while Pu6+ forms carbonate complexes (Aston, 1980). Therefore the mobility of Pu may depend on the abundances of dissolved organic compounds (fulvic acids) and on carbonate content. The presence of Cl- in surface waters may increase the mobility of Au and Ag through complex formation (AuCl2-, AgCl2-).

The presence of carbonate ions increases the mobility of U through formation of carbonate complexes. ( UO2(CO3)22-, UO2(CO3)34-, UO2CO30).