GEOTECHNICAL ENGINEERING

What is Geotechnical Engineering?

When is a rock engineering design acceptable?

Into:	-	no simple rules for acceptability

- No standard factors of safety

- Each design is unique – depends on set of circumstances e.g. rock types, design loads and end uses.

Responsibility of Geotech Engineer

- Find safe and economic solution compatible with all the constraints that apply to the project.

- Such solutions both on engineering judgment guided by practical and theoretical studies viz stability or deformation analyses.

Tables 1-4 Summarizes	 some of the

- Typical problems

- Critical parameters

- Analysis methods

- Acceptability criteria applying to different rock engineering criteria.

Purpose:	Demonstrate the diversity of problems and criteria which have to be considered and to emphasize the dangers of attempting to use Standard factor of safety or other acceptability criteria.

Ground Water Flow, Permeability and Pressure

Done:	Demarcated effect of H20 upon mining programmes

Ground water flow in Rock means

Obtain data in 2 possible ways

- Deduction of overall groundwater flow pattern from consideration of permeability of rock mass and sources of ground water.

- Direct measurement of 9-water levels in boreholes or wells or water pressure by means of piezometers.

Fig: a

Definition of permeability in terms of Darcy’s Law

Using Darcy’s Law

Q =

Where 	Q= amount of water flowing through the sample in unit time.

K = Coefficient of permeability

A = Cross sectional area of the Sample

(h1-h2) = is the difference in water table elevation between the ends

of the sample

l = length of the sample

|  | TABLE V – PERMEABILITY COEFICIENTS FOR TYPICAL ROCKS AND SOILS |  |  |  |
| --- | --- | --- | --- | --- |
|  | K – cm/sec | Intact rock | Fractured rock | Soil |
| Practically impermeable | 10-10 10-9 10-8 10-7 10-6 10-5 10-4 10-3 10-2 10-1 1.0 101 102 | Slate Balomite Granite | Clay-filled Joints Jointed rock Open-joined rock Heavily fractured rock | Homogeneous clay below zone of weathering very fine sands, organic and inorganic slits, mixtures of sand and clay, glacial till, stratified clay deposits. Clean sand, clean sand and gravel mixtures Clean gravel |
| Low discharge poor drainage |  |  |  |  |
| High discharge free drainage |  |  |  |  |

Darcy’s Low again

K =  =

V= discharge velocity units of K are vitality i.e. length per unit = time

Common to use meters/sec.

Typical layers of permeability are given in Table V.

Fig: b

The total head h can be expressed in terms of the pressure P or the end of the sample and the height z above the reference datum.  Hence

h =			Yw = density of water

From Fig a, h is the height to which the water level rises in a borehole stand pipe.

Permeability of Jointed Rock

From Table V – permeability of intact rock is very low and hence poor drainage and low discharge would usually be expected in such material.

On other hand permeability high in discontinuous rock

- Joints, fissures + other discontinuities

- high k	channels or water flow

Flow of water through fissures        Big subject:

Ref:	Huitt, J.L. “Fluid flow in simulated fracture.”

Journal, American Inst. Chemical eng vol. 2 1956 pp 259-264

Snow, D.T., Louis, C. 1968 & 1969

Simplify to

Determination of the equivalent permeability of a planar array of parallel smooth cracks.

K parallel to the array is given by

K = 		Where g = graubitural acceleration 981 cm/sec2

e = opening cracks or fissures

b = spacing between cracks and

V = coefficient of kinematic viscosity

(0.0101 cm2/sec for pure water @ 20oC)

Fig b   Shows that the k of a rock mass is very sensitive to the opening of  discontinuities.  Since the opening change with stress, the k of a rock mass will:- be sensitive to stress.

Louis 	k =  applies only to laminar flow through planar parallel fissures, this does not apply in turbulent flow i.e.

- high velocity

- rough fissure surfaces

- fissures in infield

NB: Louis equation gives highest equivalent permeability coefficient.

Lowest equivalent permeability coefficient given by

K =  (for infilled fissure system)

Where:

kf = permeability coefficient of the infilling material

kr = permeability coefficient of the intact rock.

FLOWNETS

Graphical representation of groundwater flow in a rock or soil mass is known as flownet.

Several Fetures:

Bernoulli’s Theorem of water flow: is soils deepage velocities are normally so small, the velocity head can be neglected.

Define Water Table W.T.

All soils are permeable H20 being free to flow through the interconnected pores between solid particles.

“The pressure of the pore water is measured in relation to atmospheric pressure and the level at which the pressure is atmospheric (i.e. zero) is defined as the water table (W.T) or the phreatic surface.

Perchea WT

Artesian (under pressure)

FLOW LINES:    are paths followed by the water in flowing through the

saturated rock or soil.

Equipotential lines:  These are lines joining points at which the total head

is the same.

Fig c

Water level is the same in boreholes or standpipes which terminate at points A and B on the same equipotential line.

Permeability

Example of Eq 33

[33]  K = 		g= gravitational acceleration 981cm/sec2

e= opening of cracks or fissures

b= spacing between cracks

v= the coefficient of kinematic viscosity

(0.0101 cn2/sec for pure H20 @ 20oC)

FIG d

Joint spacing e1=0.10cm

Spacing b1 = 1m

Equivalent permeability K1||to the joints is K1 = 8.1 x 10-2 cm/sec. Minor joing set has a spacing b2 = 1 joing per meter and an opening

e2 = 0.02cm.

The equivalent permeability of this set is K2 = 6.5 x 10-4 cm/sec i.e. more than 2 orders of magnitude smaller than the equivalent permeability of the major joint set.

Conclusion:  Clearly the ground water flow pattern and the drainage characteristics of a rock mass in which these two joint sets occur would be significantly influenced by the orientation f the joint sets.

One point to keep in mind when using these graphs is that the depth of the tension crack is always measured from the top of the slope as illustrated in Figure 62b-.

Consider the example in which a 100 foot high slope with a face angle of – 60o is found to have a bedding plane running through it at a dip  p = 30o.  a tension crack occurs 29 feet behind the crest of the slope and, from an accurately drawn cross-section of the slope, the tension crack is found to have a depth of 50 feet.  The unit weight of rock y = 160 ib/ft3, that of water is Yw = 62.5 ib/ft3.  Assuming that the cohesive strength of the bedding plane c = 1000 ib/ft2 and the friction angle   = 30o, find the influence of water depth zw upon the factor of safety of the slope.

The values of P and Q are found from Figure 64, for z/N = 0.5 to b:

P =1.0 and Q = 0.36

The values of R (from equation 52) and S (from Figure 64b), for a range of values of zw/z, are:

zwz		1.0		0.5		0

R		0.l95		0.098		0

S		0.26		o.13		0

The value of 2c/yM = 0.125

Hence, the factor of safety for different depths of water in the tension crack, from equation 48, varies as follows:

z/z		1.0		0.5		0

F		0.77		1.10		1.34

These values are plotted in the graph in the margin and the sensitivity of the slope of water in the tension crack is obvious.  Simple analyses of this sort, varying one parameter at a time, can be carried out in a few minutes and are useful aids to decision making.  In the example considered, it would be obviously worth taking steps to prevent water from entering the top of the tension crack.  In other cases, it may be found that the presence of water in the tension crack does not have a significant influence upon stability and that other factors are more important.

Graphical analysis of stability

As an alternative to the analytical method presented above, some readers may prefer the following graphical method:

- from an accurately drawn cross section of the slope, scale the lengths

M, X, D, A, z and zw shown in Figure 65a.

- calculate the forces W, V and U from these dimensions by means of the equations given in Figure 65a.  also calculate the magnitude of the cohesive force A.C.

- construct the force diagram illustrated in Figure 65b as follows:

DIAGRAMS

- draw a vertical line to represent the weight W of the sliding wedge.  The scale should be chosen to suit the size of the drawing board used.

- At right angles to the line representing W, draw a line to represent the force V due to water pressure in the tension crack.

- Measure the angle p as shown in Figure 65b and draw a line to represent the uplift force U due to water pressure on the sliding surface.

- Project the line representing U (shown dashed in Figure 65b) and, from the upper extremity of the line representing W, construct a perpendicular to the projection of the U line.

- From the upper extremity of the U line, draw a line at an angle   to intersect the line from W to the projection to the U line.

- The length f in Figure 65b represents the frictional force which realists sliding along the failure plane.

- The cohesive resisting force A.c can be drawn parallel to f.  although this step is not essential, drawing A.c on the force diagrams ensures that there is the error in converting to and from the various scales which may have been used in this analysis since it provides a visual check of the magnitude of A.c.

- The length of the line marked 5 on the force diagram represents the total force tending to induce  sliding down the plane.

- The factor of safety f of the slope is given by the ratio of the lengths (f + A.c) to S.

An example of the application of this graphical technique will be given later to this chapter.

Influence of groundwater on stability.

In the preceding discussion it has been assumed that it is only the water present in the tension crack and that along the failure surface which influence the stability of the slope.  This is equivalent to assuming that the rest of the rock mass is impermeable, an assumption which is certainly not always justified. Consideration must, therefore, be given to water pressure distribution other than that upon which the analysis so far presented is based.

The current state of knowledge in rock engineering does not permit a precise definition of the groundwater flow patterns in a rock mass.  Consequently, the only possibility open to the slope designer is to consider a number of realistic extremes in an attempt to bracket the range of possible factors of safety and to assess the sensitivity of the slope to variations in groundwater conditions.

- Dry Slopes

The simplest case which can be considered is that in which the slope is assumed to be completely drained.  In practical terms, this means that there is no water pressure in the tension crack or along the sliding surface.  Note that there may be moisture in the slope but, as long as no pressure is generated, it will not influence the stability of the slope.

Permeability

INSITU METHODS

Lab method reliability depends on extent to which the soil test specimens or representations of the in-situ soil means as a whole.

For important project, the in-situ determination of permeability may be justified.

- Well pumping test-used is homogenous course of strained strata

- Continuous pumping – observing water levels in a number of adjacent boreholes.

- Continue pumping until steady seepage conditions become established.

- Section Across Boreholes

Theory analysis based on assumption that the hydraulic gradient at any distance from the well is constant with depth and is equal to the slope of the water table.

i.e. in

where h = height of the water table or radius f.  this is known s the Dupuit assumption and is reasonably accurate except at point to the well.

At dist. R from the well, the area through which flow takes plafe is 2trh

Apply Darcy’s Law

or K =

This equation is applied to each pair of boreholes an average value of K is obtained.

Other insitu Methods

- Constance hence

- Variable head borehole TESTS.

Constant Head

- Allow H20 to flow under constant hence (into or out of) the stratum under test through the bottom of the borehole.

- Lower end of borehole of not less than 5d from either top or bottom of stratum.

- Maintain water level in borehole constant.

- Difference between this level and the water table level being h

- Rate of flow of required to maintain the constant H20 level is measured

Coeff. Of permeability K =

Developed from electrical analogy experiments.

Note:	ensure no dogging of soil face at the bottom of the borehole.

If deserved, H2O may be pumped into borehole under pressure.

Variable Head TEST	(2 Cases)

Measure rate of flow from stratum into borehole by observing time (t) for water level in the borehole, relative to the water table level, to change from h1 to a value h2.

Hvvasler, M.J. (951) “Time lag and soil permeability in Grand Water Observation, Bulletin NO. 36, Waterways Experimental Station, V.S. Corps of Engineers, Vicksburg, Mississippi.

Cased Borehole:	internal diameter d

Penetrating a short distance D (not exceeding 1.5m) below

the WT in a stratum assumed to be of infinite depth.

K=  in

Second case

Borehole with cased length and an uncased length or perforated extension of length L where L > ed in stratum assumed to be of infinite depth. K for this situation is given as

K =  in  in

Course Grained Sorts:

K can also be obtained form insitu measurements of seepage velocity using v1 =		n= porosity

I = hydraulic gradient (head/length

Seepage takes place from A to B

Hydraulic gradient is given by the difference in the steady state water levels in boreholes divided by the distance AB.

Use dye or other suitable tracer in borehole A and measure time taken for dye to appear in borehole B.

Seepage Velocity in then

Porosity of soil can be determined from density tests then

K =  	where 	v = seepage velocity

n = porosity

t = hydraulic gradient

porosity – ratio of volume of voids  to the total vol of the soil i.e.

n =

MI 561 GEOTENICAL ENGINEERING

Basil Mechanics of Slope Failure

Continuous approach to slope stability

Friction cohesion and density – important material properties and most relevant to slope stability

Definition-diagram below

Relationship between the shear stress is required to cane solidity along a discontinuity and the normal stress acting across it.

Relationship between shear and normal stresses for a typical rock behavior or soil sample

……………………………………………[1]

Table 1:	Typical values for angle of Friction and cohesion which are found in shear tests on range rocks and soils.

Sliding due to a gravitational loading

Consider a block of weight W resting on a plane surface inclined at an angle     to the horizontal.

Block weight W acts vertically downwards due to gravity resolve part of W which acts down the slope and which causes the block to slide is

Competent of W which acts across the plane and which tends to stabilize the block is

Normal stress acting across the potential sliding surface is given by

---------------------[2]

where A = base open of the block

Assuming that the shear strength of this surface is defined by eq. [1] and substituting for the normal stress from eq. [2]

Or  tan   ………………….[3]

where R = cA is the shear force which resists sliding down to plane limiting equilibrium (when the disturbing force is exactly equal to the resisting force.

-   -   -   - [4]

If C = 0, the condition of limiting equilibrium supplies to:

or

-------------------------------------------------- [5]

Influence of Water Pressure on Shear Strength

Open beer can filled with water on inclined piece of wood.

Condition same as above. However, assume that c c=0 i.e cohesion between bear can case and wood is zero. According to equation 5 above the can and contents will slide down the plank when .

Puncture can so that water can enter gap between plant and can giving rise to a water pressure in or an uplift force U = uA where A = base area of the can

Normal force   istnow reduced by uplift force U and the resistance to sliding is now:    -    -    -    - [6]

Where: =  wgt per unit volume of can + water

= wgt per unit volume of water

Then  and

-------------------------------- [7]

Substituting 7 into 6 gives us:

[8]

And the condition for limiting equilibrium defined in eq. 4 becames:

[9]

Assuming friction angle of the Can/wood interface is 30o, the Unpunctured can will slide down when the plane is inclined at   (Eqn 5).

The Punctured can      slide at much smaller inclination because uplift force U has reduced the normal force and hence reduced the frictional resistance to sliding.

Total weight of the can plus water is only slightly greater than the weight of the water.

Assume       and

From eq. [9], punctured can will slide when the plane is included at

The Effective Stress Law

The normal stress b acting across the failure surface is reduced to the effective stress by the water pressure u. relationship between shear strength and normal stress defined by eq. [1]

Now becomes 			[10]

NOTE: in most hard rocks, many sand soils and gravels C and  not significantly altered by presence of water hence reduction in shear strength is mainly due to reduction of normal stress across failure surfaces.

Therefore, water pressure important than moisture in defining strength characteristics of hard rocks, sands and gravels.

In terms of stability of slopes; in these materials; the presence of a small vol of H2O @ high pressure, trapped in rock mass is more important than a large volume of water discharging from a free draining aquifer.

In case of Soft rocks e.g. mudstones and shells and also in case and also in case of clays, both cohesion l and friction  can change markedly with changes in moisture content and it is necessary when testing these materials to ensure that the moisture content of the material is as close as to that existing in the field eq. 10 applies here too but note that C and  change.

The Effect of Water Pressure in a Tension Crack

Block resting on inclined plane

Assume block split by tension crack filled with H2O pressure in tension crack increases linearly with depth and a total force V due to this water pressure sets down the inclined plane in the rear race of the block.

Assume H2O pressure transmitted across the intersection of the tension crack and the base of the block, water pressure distribution in the base of the block will be as illustrated above.

This results in uplift force U-which reduces the normal force acing across this surface.

Condition of this equilibrium for this case; block acted upon by water forces V and U in addition to its own weight W is defined by

Wsin

-  -  -  - [11].

From this eq. can be seen that:

- Disturbing force downwards the plane is increased

- Frictional force resisting sliding is decreased, and

- Hence both V and U result in decreased stability.

Note:	though H2O pressures involved are relatively small; these pressures

act over large areas and hence the water FORCES can be very large.

In practice; presence of water in slope giving rise to uplift forces and water forces in tension cracks is found to be CRITICAL in controlling the stability of slopes.

Reinforcement to prevent sliding

The most efficient means for stabilizing blocks or slabs likely to slide down inclined discontinuity surfaces are:	(i)	Tensioned rock bolts on

(ii)	Cables.

Consider block above

Forces uplift force U

Water pressure force V

Rock bolt tensioned to load T inclined  at angle  to plane as shown.

Resolve components of tension bolt T parallel the plan =

Resolve components acting across surface upon which block rests =

The condition of limiting equilibrium for the case is defined by

-  -  -  - [12]

Eq. shows following:

- Bolt tension reduces disturbing force acting down plane and

- Increases the normal force and hence the frictional resistance between the base of the block and the plane.

Minimum bolt tension required:

Re-arrange eq. 12 to give expression for bolt Tension T and then minimizing this expression w.r.t the angle ,    i.e. set

Set   = 0 which gives 				[13]

FACTOR OF SAFETY

All equations defining stability of a block in an inclined plane have been presented for the condition of limiting equilibrium

i.e. the “CONDITION AT WHICH THE FORCES TENDING TO INDUCE SLIDING ARE EXCETILY BALANCED BY THOSE RESISTING SLIDING”

Factor of safety:  An index used to compare the stability of slopes under conditions other than those of limiting Equilibrium.

Define a ratio of total forces available to resist sliding to the total forces tending to induce sliding.

In other words F.S =

Considering block with Tension crack, water forces and stabilized by tensioned bolt, F.S is given by 	 -  -  - -(14)

When F = 1	 Slope is on the point of failure, a condition of limiting equilibrium exists in which the resisting and disturbing (driving forces are equal).

When slope is stable:  	resisting forces are > the disturbing forces and F > than unity.

Practical application

Eq. 14 Increase F.S by reducing both U and V   drainage or by increasing the value of T by installing bolts or tensioned cables.

Change W: evaluate carefully since both disturbing and resisting forces are decreased by a decrease in W.

Practical Experience refer to into notes

Temporally slopes 	F.S. = 1.3

Permanent slopes	F.S = 1.5

Consider rate of change of the F.S.

“The function of the design engineer is not to compute accurately but to judge soundly”

Design for worse conditions e.g.	(i)	Lots of Water

(ii)	Lower values of C and Φ

Where not clear or in virgin ground.  Learn from experience.

Slope Failures for which F.S can be calculated

Block sliding simplest possible model of rock slope failure.  In practical cases, more complex failure processed have to be considered.

NB: Drive mathematical expressions to quantify failure.

DIAGRAMS

Plane failure

- Rare sight

- Conditions rare to fulfill (Geometrical)

- Wedge type much more general case

- Useful in demonstrating the sensibility of slope failure to changes in shear strength and ground water conditions.

General conditions for plane failure:

Single plane sliding – satisfy the following:

- Plane or failure to strike 11 or near parallel (apex)

- Failure plane must “day light” in slope face i.e its dip must be smaller than that of the slope face.

DIAGRAMS

- Dip of failure plane must be greater than the angle of internal friction of the plane i.e. ψp>Φ

- The release surfaces which provide negligible resistance to sliding must be present in the rock mass to define the bilateral boundaries of the slide.

Plan failure Analysis

2 cases must be considered

- If slope is having a tension crack in its upper surface.

- If slope with a tension crack in its face.

Transition from one to the other occurs when the tension crack collides with the slope face i.e. when Z/H = (1-                 ) - - - - [1]

DIAGRAMS

Assumptions

- Both failure planes and  surfaces and tension crack strike parallel to the slope surface

- Tension crack vertical and filled with H2O to a depth Zw

- H2O enter solid surfaces alone, base for the tension crack and sips along the sliding surface – exceptional @ atmospheric pressure where the sliding surface day lights in the slope face.  [Pressure _________ as indicated in figure)

- The forces, 	W (weight of sliding block)

U (uplift force due to water pressure)

V (force due to water pressure in the tension crack

All act through the centroid of the sliding rock mass i.e. no movement to cause rotation of block and hence failure is by sliding only.

- Show strength of the sliding surface is defined by cohesion C and the friction angle Φ as related by T = c + b tan Φ

diagram 63

- Slice of unit thickness is considered and it is assumed that release surfaces are present so that there is no resistance to sliding at the lateral boundaries of the slope.

F.S  Same as for block sliding done before

F.S = cA + (Wcosψp – U – Vsinψp) Tan Φ

W. Sinψp + Vcosψp

Where from fig. 2

A = (H-Z), cosec ψp -  -  -  -  -  -  -  -  -  -  - [3]

U = ½    w.  Zw (H2). Cosec ψp - - - - - - - - [4]

V = ½   w. Z1w - - - - - - - - - - - - - - - - - - - [5]

From the tension crack in the upper slope surface (Fig 1)

W = ½    H2 (1-  Cotψp – Cot ψf - - - -  [6]

and for the tension crack in the slope face (fig 2).

W = ½    H2 (1-  Cotψp (Cotp Tanψf -1]- - - - -  970

When geometry of slope is known and Zw in the tension crack are known ____ of F.S sin simple.

However: desirable to compare a range of slope geometrics, water depths and the influence of diff. shear strengths for the cases, solutions to eqn 2-7 can become quite ____________.

In order to avoid this, re-arrange eqn2 as follows:-  (aimensunless form).

F.S = (2c/  H).  P + [Q.cotψp – R (P + S) Tan Φ	- - - - - - -[8]

Q + R.S Cot ψ

Where P = (1 – 1/H).  cosec ψp - - - - - - - - - - - - - - - - - - - [9]

When Tension crack is in the upper slope face:

Q = [(1- (2/H2) Cot ψp – Cot ψf] Sin ψP - - - - - - - - - - - - -[10]

When the tension crack is in the slope face:

Q = [(1-2/H)2 Cos ψp (Cot 4p . Tan ψf – 1)] - - - - - - - - - - [11]

R =   - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -[12]

S = .  Sin ψp  - - - - - - - - - - - - - - - - - - - - - - - - - - - [13]

The Ratios P. Q, R and S are all dimensionless-meaning they depend upon slope geometry and not upon size of the slope.

In cases where C = 0, F.S is independent of the size of the slope.

Principle or dimensionless grouping is a useful tool in rock engineering.

Facilitation of these equations to practical problems .

(Fig 64) pages 155 and 156.

NOTE:	After both tension crack positions are included in the graphs for the ratio Q and hence the values of Q may be determined for any slope configuration without having first to check on the tension crack position.

Always remember that:  depth of the tension crack is  measured from the top of the slope.  e.g.

NOTE:	Sensitivity of the slope to water in tension crack is obvious.

Investigate influence of other parameters by varying them.

In this example, it is obviously worth taking steps to prevent water

entering the top of the tension crack.

Examples of Calculation of F

F2 = (2c/  H). P + (Q cot42 – R (P + S) Tan ψ

Q +.S cot ψp

@  = 1

F = (0.125.(1) + (0.36 x 1.7321 – 0.195 (1 + 0.26) x 0.5774

=0.36 + 0.195 x 0.26. x1.7321

= (0.125 + (0.624 – 0.246) 0.5774

Tan 30o = 0.5774

Cot 300 = 1.732

0.36 + 0.0878

= 0.2904

0.4478

0.648

DIAGRAMS

Graphical Solution

This is an alternative.

Procedure:

- From an accurately drawn cross-section of the slope, scale the lengths H, X, D, A, Z and Zw shown in figure.

- Calculate the forces W, V and U from theory.

Figure :  	wgt of sliding medge

W = ½    (HX – DZ)

Horizontal ratio force:  V = ½     Zw2

Uplift force U = ½     Zw.N

Also calculate the magnitude of the cohesive force A.C.

- Construct the force diagram s illustrated and as follows:

- Draw a vertical line to represent the weight W of the sliding wedge.  The scale should be chosen to suit the size of the drawing board used.

- At high angles to the line representing W, draw a line to represent the force V due to water pressure in the tension crack.

- Measure the angle ψp as shown in FIGURE and draw a line to represent the uplift force U due to water pressure on the sliding surface.

- Project the line representing U (shown dashed in figure) and from the upper extremity of the line representing W, construct a perpendicular to the projection of the V-line.

- From the upper extremity of the U line, draw a line at an angle Φ to interest the line from W to the projection of the line.

- The length f in the figure represents the friction force which resists sliding along the failure plane.

- The cohesive resisting force A.c can be drawn parallel to f.  although this step is not essential, drawing A.c on the force diagram ensures that there is no error in converting to and from the various seals which may have been used in this analysis since it provides a visual check of the magnitude of A.c

- The length of the line marked (S) on the force diagram represents the total force tending to induce sliding down the plane.

- The factor of safety F of the slope is given by the ratio of the lengths f+A.c to s.

Influence of groundwater on stability.

DIAGRAMS

Assumed water only in Tension Crack     implying the rest of the rock as impermeable     not always true.

Currently state of knowledge in rock Engineering does not permit a precise definition of groundwater flow pattern in a rock mass.

- causes sensitivity of slope in variation in groundwater condition.

- Day Slopes	-	Simplest case

-	there may be moisture in slope, but as long as there is

no PRESSURE (water) stability will not be affected.

Under these conditions, the forces V and V are both zero:

Equation [ F = c.A + (W COSψP – U –V.Sin ψp) Tan Φ  [42 in text0

W.  Sin ψp + V.Costψp  - - - - - - - - - - - - - --  [14]

Becomes or

Reduces to:		F =  + Cot4p. Tan Φ

Alternatively equation [(8)] reduces to [ 48 in TEXT]

F = .  + CotψP Tan Φ   - - - - - - - - - - - - - - - - - - - - - - - - - -  [15]

- Water in Tension Crack Only

Heavy rain storm – fills up crack only, damage not established – therefore immediately after rain storm no uplift pressure present.  Uplift force U =0.

Uplift force – reduce to zero or nearly zero if failure of surface was impermeable as a result of clay filling.

In either case F.S of the slope would be given by:

F. C.A + (W cosψp + V cos

W Sin ψP + V cos ψP

on alternatively

F=2C/rH. P + (Q cot ψP - RS) Tan Φ

Q + R.S cot ψP

- Water in Tension Crack and on sliding surface

These are the conditions assumed in delivering the general solutions presented here.  Assumed ________ is reasonable.

Possible that a non dangerous water distribution could exist especially in frozen conditions.  Keep these possible variation in mind.

- Saturated slope with heavy recharge.

If rock mass heavily fractured     becomes relatively permeable, a ground water flow pattern as occurs in a porous system would develop. i.e.

DIAGRAMS

Equipotential distributions in slopes with various permeability configuration

The most dangerous conditions which would develop in this case would be those given by prolonged heavy rain.

How nets used for saturated slopes

F.S for permeable slope can be calculated from equation 2 and 8 assuming that the tension crack is water-filled Ze Sw = Z

- Uncertain – the associated with the actual water pressure distribution in rock slopes. No need for further detailed analysis

Critical Tension Crack Depth

- position of T.C may not be known due to = presence of a waste dump for e.g.

Fig 66  Influence of tension crack depth and depth of H2O in Tension Crack

upon the factor of a slope.  (from example solution).

When slope is dry or nearly dry – F.S reaches _____________ value which in

example considered corresponds to a tension crack depth of 0.42H.

find the by minimizing r.h. side of following equation w.n.t

F.S =  + cotψP.Tan Φ		9

Critical tension crack depth as:

Zc/H = 1 -  - - - - - - - - - - - - - - - - - - - - - - -  [16]

From the Geometry of the slope, the corresponding position of the tension crack is

bc/H =  - cot 4f

Critical tension crack depths and locations for a range of dry slopes are in future 67.

DIAGRAMS

Depth and location of the tension crack are however independent of he groundwater conditions and are as defined by eqn3 55259

= 1 -  - (55) - - - - - - - - - - - - - - - - - - - - - - - -  [20]

=   - (59) - - - - - - - - - - - - - - - - - - - - - - - - - - - - [21]

Equation 58 and 59 no longer apply in a newly formed tension crack as those that form after a heavy rain storm

DIAGRAM

Figure 66:	Influence of tension crack depth and of depth of water in the

tension crack upon the factor of safety of a slope.  (slope geometry and material properties as for example on page 154)

THE TENSION CRACK AS AN INDICATOR OF INSTABILITY

- Tension cracks are frequent occurrences

- Can exist for years (observation)

Barton   Tension crack generated as a result of small shear movements within rock mass.

- Small movements but numerous          significant displacement of slope faces

- Fact tension crack caused by shear movements means that when tension cracks appear, it must be assumed there shear failure has initiated in the rock mass.

- Not possible to quantify ___________ of this failure.

Recommendation

- Presence of T.C to be taken as an indication of potential instability.

- Presence should signal the need for detailed investigation into the stability of the slope.

Critical Failure Plane Inclination

How to determine when not visible or indicated?

In soft rock or soil slope __________ slope face i.e. ψP <450.  The failure plane would be ____

In sleep rock slopes, failure surface is almost planar

Indication of such plane can be found by partial differentiation of Eqn 42 wnt ψp

Then F.S = _CA + (Wcon4p – u-v sin 4p) Tan Φ

W. sin 4p + V cos 4p

and by equating the resulting differential to zero.  For dry slopes, the critical failure plane inclination ψp is

ψp  = ½ (ψp + Φ)  - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -  [17]

presence of H2O in slope will cause the failure plane inclination to be reduced by up to 10%.

DIAGRAM

- Slope failure before left tension crack Z1, left a face included at ψf

- As a result m – undercut DM inclined at angle ψo, a new failure across on a plane inclined at ψp and involves the formation of a new tension crack of depth Z2.

Geometry of undercut Slope

Not unusual for a tie of a slope to be underwent.

- Intentionally

- Natural agencies/weathering of underlying state.

- Sea cliffs    by action of waves, epision.

42

Influence of undercutting on slope stability is important.

F.S given by eqn 	modified for the weight term as follows:-

Note that for ψo>0

DM = (H2 – H1)Cot ψ0

The critical tension crack depth, for  a day under-cut sloe is given by:

Z2 = C. Cos Φ

Cos ψp. Sin (ψp – Φ) -----------------------------------------------[18b]

The critical failure plane inclination is:-

P = ½ (Φ + Arctan

Reinforcement of a Slope

DIAGRAM

- Establish instability

- Consider stabilizing by 	-	drainage

-	application of external loads

External loads 	Application of installation rock bolts or c_______

__________ into the rock.

Construction of a waste rock _____ to support the toe

of the slope.

The F.S of a slope with external loading of magnitude T inclined at an > 0 to the failure plane as shown in the sketch above is

The F.S = C.A (W/cos ψp – U - VSin ψp + T. cos ) TanΦ

WSin ψp + V cos ψp + T. cos ) – T.Sin  ------------------------------ [25]

Equation I, correct for the condition of limiting

Problems in using it for other values of F.S (these problems are discussed fully in Appendix 3).

Analysis of failure on a rough plane

Most rocks surfaces exhibit a mass-linear relationship between shear strength and effective normal stress.

The relationship is defined by Ladanyi and Anchambants’ equation

L= (l-as) (v + Tan Φ) + as.tr

1 – (1 – as) v Tan Φ

Where: as 	is the proportion of the discontinuity surface which is sheared

through projection of intact rock material

v 	is the dilation rate  at peak shear strength and

Tr	is the shear strength of the intact material

OR  but Barton’s equation

l =  Tan (Φ JRC. Log10          )

where JRC is the joint roughness coefficient which is defined as in Figure

below.

Examples of Roughness Profiles

500m                                                        500m

A:	Rough undulating – tension joins, 				JRC = 20

B:	Smooth undulating, smooth sheeting 			JRC = 10

hm planar foliation, undulating loading

C:	Smooth nearly planar – planar shear joints, planar

foliation, planar bedding  					JRC = 5

= Joint Compressive strength

= Effective normal stress.

DIAGRAMS

Effective normal stress acting on the failure surface can be determined from Equations 3 – 7 notes

43 – 52 text

b = (Q cot 4p – R (P+S)

26

21

having determined the value of    , the shear strength L of the failure surface is calculated from equation 	  on

Text		Text

Barton’s equation

Ladanyi and

Archandanlts

The factor of safety of the slope is given by modifying equation

8

2

142

148

and 	             as follows:-

Text		     Text

F.S  ----------------------------------------------------------------[23]

OR

F = 	    2 P.I

H (Q + R.S. CotψP ----------------------------------------------------------[24]

Application  Best Illustration

Chalk Cliff failure @ Joss Bay Iste of Thanet, England.

- Failure indirect by undercutting action of the sea

- Failure in cots fig 87

- Cross-section of cliff.

- H2O measurements 	-	fully drained

- No heavy rain fall

- Water filled tension crack:-  empty not considered in analysis (though interest can check on route influence.)

Laboratory tests on Cliff face samples

density 1.9 tonnes/m3

friction angle Φ 42o for peak strength

30o for residual strength

Cohesive strength C range 13.3 tonnes/m2 peak

0 tonnes/m2 residual

Purpose:  	Determine the shear strength mobilized in the actual failure and to 		compare this with the lab values.

Summary of available input data:

H – Slope height (H1 = H2)		15.4m  (no tension crack considered)

Z1 – Original tension crack depth 	6.8m

Zz – New tension crack depth 		7.8

DM – depth of undercut			0.5m

Ψ0 – inclination of undercut		0o

Ψf – Slope face angle			80o

ΨP – failure plane angle			67o

[a]	The effective friction angle of the chalk mass can be determined by re-arranging equation [19] notes

[64] Text

Φ = 24p – Arcton

_____________ give Φ=49.9o – Note significantly higher than the friction angle measured is lab – 42o

-	_____ influence of the roughness of the 						actual failure surface should be taken into

account in comparison.

From ____ - note that face is very rough – diff in Φ not surprising.

[b]	The cohesion ___________ @ failure can be established by rearranging 	equation [186] Notes

[63  ] Text

C=

Sus___________ Z2 = 7.8m, ψp = 67o and Φ = 49.9o gives C = 2.64 tonnes/m2

[i]	What will happen if the Joss Bay Cliff conti.  It is instructive to consider what will happen to the Joss Bay Cliff as the sea continues to undercut in to Influx data for this next step in the failure process as follows:

H = Slope height (H1 – H2) = 154m

Z1 = Original tension crack d = 7.8m

Ψf = Slope face angle	        = 67o

C = cohesive strength of the chalk mass 2.65 tonnes/m2

Φ = friction angle of chalk mass 49.9o

The unknown in this analysis are:-

Z2 = New tension crack depth

Ψp = failure plane angle

DM = depth of undercut

Note:  3 unknowns and only 2 equations viz recall

[65] Z2 =          and

Ψp = ½ (Φ + Arctan

Obtain solution in the following matter:

- From eqn b	(i)	calculate depth of tension crack Z2 for a range of

possible failure plane angles ψp

(ii)	Plot as in figure (90)

- Since Z2 must lie between Z1 and H1 fig 90 shows that the angle of the failure plane ψp must lie between 67o and 56o.

DIAGRAMS

Solving for a range of corresponding values of ψp and Z2 gives the depth of the undercut shown in Fig. 90b.

Clear that:

- Further cliff failure will occur when undercut reaches a depth of approx. 0.9m, and that two corresponding failure plane angle will be ψp = 60o and new tension crack will be (Z2) = 10.2m.

Consequences:  serious for property owners on cliff tops cliff failure: -

Consider stabilization

Analysis assumed chalk mass dry

Presence of H20       serious reduction of face stability

Steps to stabilize (i)  drainage 	-	complete

-	no pools of water

-	horizontal ____________ into

face to allow free drainage.

Fact	Stability of this slope extremely sensitive to u/cutting.

Prevent by provision of concrete wall along the toe of the cliff – may be

practicle solution (in some cases) – 	may be impossible to provide secure foundation for such a wall.

If not possible

Only way is to stabilize by reinforcement material failure involved – assume 50 tones for water or slope – small stabilisation ______

Failure process distant        suggest fully grounded belts or cables, lightly ____ ____ to ensure all contacts are closed.

Load capacity of bolts should be     25% of the mass of the material which could fall.

- Use pattern of 2o tones bolts or cables each 5m long installed on 5m ____.

- If chalk mass is closely jointed and reveling in- use closer pattern, or conation free __________________________

Practical Example 2

Rock Sliding on clay layers

In mining a horizontal bedded coal deposit, sliding at blocks of material on clay occurs, during periods or high fall:  Clay seams have a high __________ content and that clay, slicken sided by previous shear displacements.

Consequently very low residual shear strength values of C = 0 and Φ = 10o are common and considered appropriate for analysis of such failures.  Ref 114.

Ref 114:  Stimpson, B. and Walton G.

“Clay mylonites in English Coal Measures”

Proc.  1st Congress, International Association of Engineering Geology.  Paris Vo. 2 1979 pages 1388-1393

1388-1393