MI 315:  ROCK MECHANICS LAB NO. 2B

DETERMINATION OF UNIAXIAL COMPRESSIVE STRENGTH OF CYLINDRICAL ROCK SPECIMENS BY INDIRECT METHOD (POINT LOAD TEST)

- Scope

This test measures the estimated uniaxial compressive strength of cylindrical rock specimens for strength classification and characterization of intack rock.

- Sample Preparation

Specimens do not need any special preparations.  Ideally the specimen should be NX size and length of at least 1.5 times the diameter.  Unlike the direct method, this test requires as many specimens as possible to determine a reasonable estimate.

- Apparatus

A point load test equipment with two hardened steel points, able to apply and measure an axial load to the specimen.  The point load tester has sufficient capacity and capable to apply a load at a constant rate.

- Procedure

- Ensure the Point load tester’s dial gauge is at zero

- Place the specimen at a point that is at least 70% of the specimen diameter from either ends of the core.

- Load the specimen continuously at a constant rate  until failure occurs.

- Record the failure load. (P)

- Repeat (i)-(iv) for the remaining specimens

- Calculations

- The Point Load Index is computed as follows;

Point Load Index, Is=P/D2

Where; P is failure load expressed as a force

D is the specimen diameter

- The Estimated Uniaxial Compressive Strength, therefore is computed as follows;

- For specimens whose D = 50 mm, ac = 241

- For other values of D,		         ac = (14+0.175D) Is

Assignment

Write a lab report and determine the Point Load Index.  Give reasons why Engineers should use the point load index to estimate the Uniaxial Compressive Strength.