# THE UNIVERSITY OF ZAMBIA

UNIVETRSITY EXAMINATION

SCHOOL OF MINES

# DEPARTMENT OF METALLURGY AND MINERAL PROCESSING

## MM 415 – Mineral Processing                                                July/August, 2001

### Attempt: Five Questions. Questions (1) and (3) are compulsory. Time: 3 hours

- State briefly but clearly what you understand by the following terms used in mineral processing:

- metallic minerals

- contained value of an ore deposit

- liberation

- middlings fraction

- 80% passing reduction ratio

- angle of “nip”

- reverse flotation

- hindered settling ratio of two minerals

- pulp density of a solid/liquid mixture

- (a) Write short but clear notes of reasonable length on:

- Jaw crusher

- Gyratory crusher

- Cone crusher

- Hardinge mill

(b) What are mill-liners and for what specific purpose are they used?

- (a) What are the advantages of wet grinding in mineral processing operations?

(b) What do you understand by open- and closed-circuit grinding? What are the                                                                                               	advantages of each?

(c) The cyclone overflow line is instrumented with a magnetic flow meter and       	nuclear density gauge, and the mass of dry ore to flotation is 25 t/hour. The 	feed from the fine ore bins is sampled, and is found to contain 5% moisture. 	The cyclone feed contains 33% solids, the cyclone underflow 65% solids, and 	the overflow 15% solids.

Required:

- circulating load on the circuit

- amount of water needed to dilute the ball mill discharge

- draw the operating flow sheet and indicate whether it is an open- or closed-circuit grinding operation.

Hint: Input to circuit = output

- Broken rock of 80% passing 2500m is ground wet in a ball mill to a product of 80% passing 225m. This size reduction required 8 kWh per tonne of feed.

- How much energy would be required per tonne to reduce this rock from the same feed size to 80% passing 100m in the same ball mill?

- If it would be required to mill 11000 t/day of this same rock from 2500m to 100m (80% passing sizes) in three (3) shift operation with an expected mill efficiency of 89%, how much energy would that require per 24 hour day?

- What minimum horsepower should be installed in the grinding section based upon the above data?

1 hp = 0.75 kW

- (a) What are the advantages of hydrocyclones over mechanical classifiers?   	Briefly describe the efficiency of a cyclone.

(b) What are jigs, and what are their main applications?

(c) What are the main requirements of the solid particles to be used as a 	medium in heavy media cyclones? Explain briefly.

- (a) What is froth flotation? Describe the role of the following chemical 	reagents in flotation.

- collectors

- frothers, and

- depressants

Give examples in each case.

(b) What is the importance of pH in froth flotation?

(c) What are the “scavenger” and “cleaner” circuits in flotation?

- (a) What are the main methods of dewatering? Briefly describe each of 	them.

(b) Describe briefly the construction of tailing dams by the upstream, 	downstream and centre-line methods. What are the advantages of each 	method? Illustrate with sketches.

END OF EXAMINATION