THE UNIVERSITY OF ZAMBIA

SCHOOL OF ENGINEERING

MECHANICAL ENGINEERING DEPARTMENT

AC AND DC GENERATORS LAB

NAME: SIKAONA NDANJI

COMPUTER NUMBER:2019090716

COURSE: MEC 3102

LECTURE: MR. BOYD MUNKOMBWE

# AIM

To investigate the variation of output voltage and waveform with field current and speed for both an A.C and a D.C generator.

# INTRODUCTION

A generator is an electro-mechanical device (electrical machine) which converts mechanical energy or power (𝜔𝑇) into electrical energy or power (𝐸𝐼)

Voltage is produced by generators due to relative motion of a conductor and magnetic flux. An average value of induced voltage VI = k n Ф is obtained by applying Faradays law v = 𝑑Ф, n is

the speed and Ф is the flux per pole which is a constant depending on machine design. Magnetic flux Ф is associated with the coil carrying the “field current”.

Any one conductor in the machine has an alternating voltage induced in it, one half cycle being produced as it passes a north pole and the other half cycle of opposite polarity as it passes a south pole. Frequency f ((Hz), thus depends on the speed of rotation (in rev/s) and number of pole parts p.

In an alternator (ac generator) the designer ensures that the arrangement of a large number of conductors in series and the shape of the magnetic circuit of the poles produce a sinusoidal voltage output with the field being provided by the rotor excited with dc current through sliprings. The large ac output power comes from permanent connections to the stator windings.

To obtain D.C from a generator it is necessary to reverse the connections to the conductor when it is changing from being under a north to under a south pole, and vice versa, so that the output voltage in the external circuit is always unidirectional. This is achieved by having a stationary field system, and the rotor conductors connected to a commutator (instead of sliprings in the a.c case) with it’s associated brush gear.

# APPARATUS

- motor

- Power source

- Voltmeter

- Ammeter

- Cables

# PROCEDURE

A.C experiment

Generator was switched on and speed was locked at 1259 rpm as the current was being increased from 0.18A to 0.28A and values of generated voltage were captured and recorded in the data collection table.

The field current was kept constant at 0.1A and speed varied from 899 rpm to  1739 rpm and the corresponding voltages were captured and recorded in the data collection tables.

DC experiment

Motor an DC generator was switched on and speed n was locked at 1259 rpm while the field current was varied from 0.06A to 0.14A and the generated voltage was recorded in the data collection.

Field current kept constant at 0.06A and speed n varied from 899 rpm to a maximum value of 2279 rpm and the generated voltage was recorded in the data collection tables.

DATA COLLECTION

- A.C

Current =0.1A	speed=1259 rpm

| speed (rpm) | Voltage(v) |
| --- | --- |
| 899 | 70 |
| 1079 | 80 |
| 1259 | 100 |
| 1439 | 113 |
| 1619 | 130 |

| Current(A) | Voltage(v) |
| --- | --- |
| 0.04 | 50 |
| 0.08 | 70 |
| 0.12 | 114 |
| 0.16 | 140 |
| 0.2 | 155 |
| 0.24 | 170 |

- D.C

Current=0.1A	speed=1259rpm

| Speed(rpm) | Voltage(v) |
| --- | --- |
| 899 | 45 |
| 1079 | 53 |
| 1259 | 62 |
| 1439 | 72 |
| 1619 | 87 |
| 1799 | 99 |

| Current(A) | Voltage(v) |
| --- | --- |
| 0.02 | 31 |
| 0.04 | 43 |
| 0.06 | 54 |
| 0.08 | 60 |
| 0.1 | 63 |
| 0.12 | 69 |

# FOR AC

# (i) when field current is constant=0.1A

# (ii) when speed is =1259 rpm

# FOR DC

# (iii) when field current is=0.1A

# (iv) when speed is =1259 rpm

# DISCUSSION

.

In the AC generator experiment when If (0.10A) is kept constant and the speed n varied, the voltage (Eg) increases gradually for the first two values and slanted sharply as the speed increased the motor.

When speed n (1259 rpm) is kept constant and the current is varied the voltage increases gradually but it was observed that the voltage produced was higher when current was varied instead of speed.

In the DC generator experiment when If (0.1A) the voltage sharply as the speed n (rpm) is varied. When speed n=1259 (rpm) is kept constant and the field current If is slowly increased the voltage values keep increasing.

It is observed that for both AC and DC where field current was kept constant and speed varied (increased) we got more voltage produced

# CONCLUSION

The experiment was a success as the  values of voltage were successfully recorded by varying speed and current in both the A.C and D.C generator.

# REFERENCES

S. K. Sahdev Electrical Machines.

V.K. Mehta, Rohit Mehta Principles of Electrical Machines

Sign………………………………